May 11th, 1994

MT stands for Money Talks. It is a DOS program designed to recognize

paper currency using a scanner. All of the Hewlett Packard scanners

are supported. If you have Reading AdvantEdge, Money Talks also

supports all of the other scanners which Reading AdvantEdge supports

(the SA4-3 BookEdge scanner, the GS-Plus scanner, as well as the SA4-3,

SA4-2, GS-Plus, and Ricoh scanners connected via the PC/KPR coprocessor

card). Paper currency of the United States and of Canada can be recognized.

Money Talks requires a PC powered by an 80386 or better processor,

containing at least 2 megabytes of memory.

INSTALLATION

------------

Place MT.EXE and the template files it came with (extension .TPL) in

a directory which is on your path. If you wish to recognize Canadian

currency by default, then copy MTCAN.TPL over the file MT.TPL.

Money Talks will search for a Reading AdvantEdge configuration file

when it runs. If none is found, it will report the following:

The Reading AdvantEdge configuration file could not be found.

Hewlett Packard IIP presumed.

Any HP scanner will work in the absence of a configuration file. If a

Reading AdvantEdge configuration file is found, that file will inform

Money Talks as to which scanner is to be used.

OPERATION

---------

Make sure your scanner is powered on. Place a bill on the scanner

near the top edge, such that a long edge of the bill is parallel to

the top edge of the scanner. You'll find that you don't have to be

too careful - the bottom of the bill can be as far away as four

inches from the top of the scanner, and MT will deskew the image.

Replace the platen cover. Now type MT. The scanner will scan, and the

denomination of the bill will be written on the screen. For example, if a

10 dollar bill was placed on the scanner, MT would write the following on

the screen:

10.

Press Q to quit, or any other key to scan again.

If you have more currency which you wish to recognize, remove the

first bill and place another on the scanner, then press any key other

than Q. The new bill will be scanned, and its denomination reported.

If you press Q, the program will exit.

Money Talks tries to be very sure of the denomination before it

reports it. If it is less than certain, it will precede the

denomination with the phrase "Probably a", like the example below.

Probably a 10.

Press Q to quit, or any other key to scan again.

In cases like this, you might like to flip the bill over and confirm

the recognition attempt.

In some circumstances, particularly if the bill is badly faded,

wrinkled, or torn, Money Talks might not be able to identify it. In

those cases, it will report the following:

Sorry, I don't know what this is!

You might try smoothing the bill, repositioning it, flipping it, and

trying again.

BELLS AND WHISTLES

------------------

Type MT ? and it will tell you about itself. It supports a number of

options which are invoked with command line switches. A switch is a

single alphabetic character on the command line preceded by a dash

or a forward slash. The following switches are supported.

-F

The -F switch stands for full page. When you specify the -F switch,

the scanner will scan 11 inches instead of 4. Multiple bills may be

placed on the scanner, as long as they are parallel with the top edge,

and as long as they do not overlap. Each will be identified and reported

in turn.

-V

The -V switch stands for verbose. When you specify the -V switch, you

will get additional information about the match. For each corner

matched, the best match will be described. The denomination, side,

and probability are displayed. The probability is between 1 and 100.

An example is below.

Best match for Top Left corner was a 1, Face side showing, probability 63.

Best match for Top Right corner was a 1, Face side showing, probability 62.

Best match for Bottom Left corner was a 1, Face side showing, probability 71.

Best match for Bottom Right corner was a 1, Face side showing, probability 76.

Probably a 1.

Press Q to quit, or any other key to scan again.

-S

The -S switch stands for single scan. When you specify the -S switch,

you will not be prompted about another page. Instead, one image will

be scanned, the bill will be recognized and its denomination

reported, and the program will exit. This mode of operation is

particularly useful if you want to embed MT in a batch file.

-T template file

The -T switch stands for template. The matching process works by

comparing properties extracted from each corner of the bill with

similar properties in a group of templates, or models. Those models

are loaded from a template file. The template file must be located in

the same directory that contains the executable file, i.e., MT.EXE.

If the -T switch is not specified, the template file used will be

MT.TPL.

Two template files come with this release of Money Talks. One is for

U.S. currency, the other for Canadian currency. The former is called

MTUS.TPL, the latter is MTCAN.TPL. You can switch between them at

will using the -T option. Of course, you can also copy either over

MT.TPL, making that particular template file the default.

-B brightness level

Money Talks uses a default brightness setting for each scanner. I

encourage you to try that first. However, all scanners are different

and you may find that you wish to experiment with a different

brightness setting. The range of brightness values differs depending

upon the scanner you use. For most Hewlett Packard scanners, the GS-Plus,

and the SA4-3, the range is 1 to 255. For the original Hewlett

Packard ScanJet, the range is 1 to 3. For scanners connected to the

old PC/KPR card, the range is 1 to 16. Lower numbers cause images to

darken, higher numbers cause images to brighten.

DIAGNOSTICS

-----------

The following diagnostic error messages may occur.

Sorry, there is not enough memory to scan a page.

MT.EXE contains a DOS extender, and is loaded almost entirely into

extended memory. Your PC must have at least a megabyte of available

extended memory. Consequently, a PC with, say, one megabyte of total

memory will not be able to run Money Talks. Even if you have a lot of

memory, you may find that your current configuration uses it in a way

that prevents Money Talks from running properly. Disk cacheing

programs like SMARTDRV use extended memory, as do RAM drives. Memory

managers like EMM386, QEMM386, and others may convert extended memory

to expanded memory, making it unavailable. If you do not need

expanded memory, use a NOEMS option for EMM386 (refer to your

documentation for equivalent options for other memory managers).

Sorry, no scanner driver was found!

If you are using an HP scanner, this indicates that HPII.SYS, or

HPIIX.SYS, or SJDRIVER.SYS is not properly loaded. Refer to your

scanner's documentation.

If you are using an XIS scanner with Reading AdvantEdge, this

indicates that the appropriate scanner driver, which could be

XISSA43.SYS, XIS380GS.SYS, or LAMPSCAN.SYS is not properly loaded.

Refer to your Reading AdvantEdge documentation.

Sorry, the scanner is not ready.

A scanner driver was found, but the scanner is in an error state, or

is not powered on, or the scanner cable is not properly connected.

Correct the situation, and try again.

Sorry, a scanner error was detected.

A scanner error occurred after scanning began. You should generally

power cycle the scanner and try again.

Sorry, nothing was found on the scanner.

Nothing that was the appropriate size for U.S. or Canadian currency

was found. If you did place a bill on the scanner, reposition it so

that it is within the first four inches of the top of the scanner

platen, and try again.

An image found appears to be the wrong width!

After eliminating faint and small images, and after deskewing an

image that remains, it was determined that the image was either too

wide or not wide enough to be a bill. This is sometimes caused by

ambient light on the scanner platen - you should close the lid on

your scanner. It can also be caused by raised markings on the edge of

the platen - scanners provided by Arkenstone have sometimes been

modified to provide more tactile feedback, but those modifications

cause more light leakage. In those cases, you might find that taping

a white sheet of paper lengthwise across the bottom of your scanner

lid might help. Finally, of course, the bill may be folded over. By

the way, the program always checks for multiple images, so it could

conceivably report this message, and then go on and recognize a

different image.

An image found appears to be the wrong height!

Once an image has been checked for width, it is checked for height.

If the height is inappropriate for a bill, this message will be

displayed. See the previous error message for further information.

Yikes - image lost after deskew operation!

The image of a bill was so faint that it was lost entirely after the

deskew operation. Reposition the bill and try again.

Yikes - image broken up after deskew operation!

The image of a bill was so faint that it was broken into multiple

images after the deskew operation. Reposition the bill and try again.

KNOWN PROBLEMS

--------------

Caveat Canadian!

Canadian currency is very colorful stuff. It makes U.S. currency look

quite plain by comparison. Unfortunately, scanners differ as to how

well they can see pastel colors. This has made identification of

canadian currency a tougher problem.

The BookEdge scanner (SA4-3 or SA4-2) does not see images which are red.

Canadian 50 and 2 dollar bills will typically not be seen at

all. Canadian 100 and 10 dollar bills frequently work on one side but

not on the other. Fewer problems have been noted with the GS-Plus or

Hewlett Packard scanners, but even with them, identification of

Canadian currency has been somewhat less reliable than identification

of U.S. currency.

SUPPORT

-------

I'll be happy to answer questions and accept suggestions.

I (Stephen Baum) can be reached on the Internet as srb@xis.xerox.com,

on CompuServe as 71102,405, on Genie as S.BAUM, on Fidonet

as 1:101/460.0, or, through snail mail, at

Xerox Imaging Systems

Attn: Stephen Baum

9 Centennial Drive

Peabody, MA 01960

LEGALISMS

---------

This is freeware. You may distribute it anywhere you wish, as long as

you do not modify it, and as long as this document accompanies the

executable. You may not sell it, although you may charge a small

price to recover the cost of the medium that you distribute it on.

MT.EXE contains the DOS/4GW Professional extender, created and

copyrighted by Rational Systems, Inc.

BookEdge and Reading AdvantEdge are trademarks of Xerox Imaging

Systems, Inc.

HP, and HPIIP are trademarks of Hewlett Packard, Inc. All other

products are trademarks of their respective owners.

Xerox Imaging Systems, Inc. disclaims all warranties as to this software,

whether express or implied, including without limitation any implied

warranties of merchantability, fitness for a particular purpose,

functionality, data integrity or protection.

CHANGES

-------

Version 1.0D was made on June 3rd, 1994. It will search for the

Reading AdvantEdge configuration file in directories on the path if

that file is not found in the directory containing MT.EXE.

Version 1.0E was made on June 8th, 1994. It links in some corrections

to the Hewlett Packard scanner code. Earlier versions misbehave on

some, but not all, processors at scanner initialization time.

Version 1.0F was made on June 21st, 1994 (summer solstice!). It

automatically detects whether the background color of the scanner is

black or white in its attempt to find the bill. This makes it more

likely to work when the platen cover is up, and allows it to function

better on the Arkenstone modified HP scanners whose platen covers do

not properly close.