This document will provide you with the proper Installation Commands for

Speech,

Braille, or both.  Sample Installation Commands will be given.

Be sure to read VOICE.DOC for instructions regarding the proper

preparation of your Synthesizer, as this may affect the Installation

Program.

**THE INSTALLATION CHECK LIST

Before installing SLIMWARE WINDOW BRIDGE, remove all other Memory-

resident (TSR) type programs from memory such as Virus Checkers,

other Screen Review, Braille Display, and Large Print programs.

These may interfere with the proper installation of the program.

The Installation program will initiate communications with your

synthesizer or braille display, and will provide you with all

necessary information throughout the installation process.

When using a Laptop or Notebook Computer, be sure to connect the

Power Adaptor before turning your PC, ON.

This will ensure that the MASTER DISK is read properly.

**SERIAL PORT ORGANIZATION

When using a Serial Speech Synthesizer, connect it to COM1 or 2.

Braille Displays can be connected to COMM 1 to 4.

If you are also using a Fax Card, Modem, Mouse, Tracking Ball, Touch

Window, or Network Card, you must be sure that the Serial Port Interrupts

do not conflict between these devices.  Serial Port 1 and 2 are the safest

Ports to use for these devices.  If you plan on using Port 3 and 4

because you have more than 2 Serial Devices, you must be sure that

the Interrupts do not conflict with other devices.

Your PC shares COM1 Interrupts with COM3, and COM2 Interrupts

with COM4.

Therefore, the general rule to follow is that you should not connect a

Serial device on COM3 if your Synthesizer is using COM1.

You should not connect a Serial Device to COM4 if your Serial

Synthesizer is connected to COM2.  If you have 3 Serial devices,

put the Mouse on COM 1, the Synthesizer on COM 2, and configure the

Modem to use a different Interrupt than COM 1, and place it on COM

3.

**WINDOWS 95 PREPARATION

If you are running WINDOWS 95, you will need to "Shut Down" WINDOWS.

You will then need to Reset your PC, and interrupt the loading of

WINDOWS 95.  This is done by pressing FUNCTION KEY #8 as soon as you hear

the Tone from your PC.  This generates a Menu on the screen.

Move Up the List with the UP ARROW until you are positioned on:

"Command Prompt Only"

Press ENTER to be taken to a DOS Prompt.

Create a Batch file called WIN.BAT in the Root Directory by

following these steps:

1. Type the following at the DOS Prompt:

Copy con win.bat

2. Press ENTER.

This will make the DOS Prompt disappear from the current line.

3. Type the following on the keyboard:

@echo off

4. Press ENTER.

5. Press FUNCTION KEY #6.

6. Press ENTER.

This will write a file called WIN.BAT that only contains a Line

of text that will stop WINDOWS 95 from automatically loading when

you turn your PC ON unless you have a Command at the end of your

AUTOEXEC.BAT file that changes to the WINDOWS 95 Directory.

If this exists, you will need to remove this line from AUTOEXEC.BAT.

*

*STARTING THE INSTALLATION PROCESS

Insert Disk #1 into Drive A: or B:.  Remove all other Screen

Review, Braille, and Large Print Programs from memory.

From DOS, log to the Drive containing the Install Disk.  Select

the most appropriate Install Command:

**INSTALLING WINDOW BRIDGE FOR A SPEECH SYNTHESIZER

When preparing to Install your SYNTHA-VOICE Product for use with a

Speech Synthesizer without a Braille Display, be sure to load any

required Speech Drivers included with your Synthesizer with the

exception of the DECTALK EXPRESS, DOUBLETALK, KEYNOTE, or SPEAK-OUT

Drivers.  These Drivers should not be used when using your SYNTHA-

VOICE Product.  Insert the SYNTHA-VOICE MASTER DISK into either

Drive A: or B:.  Change to this Drive by typing A: or B:.  Press

ENTER.  The valid Install Commands are:

INSTALL #        Replace "#" with "0" if you are using SMARTALK, or

an Internal Synthesizer that does not communicate

with your PC as a Serial device.  Otherwise,

replace "#" with "1 or 2 to identify the Serial

(COM) Port where your Synthesizer is connected.

INSTALL 0 AP     When installing an APOLLO II on LPT1.  Note, only

LPT1 is supported.

INSTALL 0 TA C:  When installing for TEXT ASSIST and a

CREATIVE LABS "SOUND BLASTER" on Drive C:.

You can replace "C:" with an alternate Hard Drive Label.

This will execute a Full Installation on the specified

Drive.  You will not be taken through the Installation

Menu because TEXT ASSIST cannot talk without WINDOWS.

This automatic Installation is only valid for TEXT

ASSIST.

INSTALL G#       When using a Generic Synthesizer such as the

original ECHO, BRAILLE N' SPEAK, or BRAILLE N'

SPEAK 640.  Replace "#" with the Serial Port used

by this device.  Valid Numbers are 1-4.

**INSTALLING FOR A BRAILLE DISPLAY

When preparing to Install your SYNTHA-VOICE Product for use with a

Braille Display, Insert the SYNTHA-VOICE MASTER DISK into either

Drive A: or B:.  Change to this Drive by typing A: or B:.  Press

ENTER.  The valid Install Commands are:

INSTALL AD/L#          When using the ALVA ABT 3-40 Desktop Braille

Display.  Replace "#" with the Parallel Port

(LPT) used by this device.  When using LPT1,

the "/L1" is not required because the

Install

Program will automatically assume that LPT1

is being used.

INSTALL A24/L#         When using the ALVA ABT 2-40 Braille

Display.

Replace "#" with the Parallel Port (LPT)used

by this device.  When using LPT1, the "/L1" is

not required because the Install Program will

automatically assume that LPT1 is being

used.

INSTALL A28/L#         When using the ALVA ABT 2-80 BrailleDisplay.

Replace "#" with the Parallel Port (LPT) used

by this device.  When using LPT1, the "/L1" is

not required because the Install Program will

automatically assume that LPT1 is being

used.

INSTALL A32/L#         When using the ALVA ABT 3-20 Braille Display.

Replace "#" with the Parallel Port (LPT) used

by this device.  When using LPT1, the "/L1" is

not required because the Install Program will

automatically assume that LPT1 is being

used.

INSTALL A32/S#/B#      When using the ALVA ABT 3-20 Braille Display.

Replace the "#" following the Letter "S" with

the Serial Port used by this device.  Valid

choices are 1-4.

INSTALL A34/L#         When using the ALVA ABT 3-40 Portable Braille

Display.  Replace "#" with the Parallel Port

(LPT) used by this device.  When using LPT1,

the "/L1" is not required because the Install

Program will automatically assume that LPT1 is

being used.

INSTALL A34/S#/B#      When using the ALVA ABT 3-40 Braille Display.

Replace the "#" following the Letter "S" with

the Serial Port used by this device.  Valid

choices are 1-4.

INSTALL A38/L#         When using the ALVA ABT 3-80 Braille Display.

Replace "#" with the Parallel Port (LPT) used

by this device.  When using LPT1, the "/L1" is

not required because the Install Program will

automatically assume that LPT1 is being used.

INSTALL A38/S#/B#      When using the ALVA ABT 3-80 Braille Display.

Replace the "#" following the Letter "S" with

the Serial Port used by this device.  Valid

choices are 1-4.

INSTALL B              To install for a supported Braille Display

without specifying a specific Model as part of

the INSTALL Command.  This will access a Menu

of available Braille Displays and available

locations for the Braille Display.  Note: You

will not receive Braille output for this Menu

until you have selected the correct Model and

location from the Menu.  Do not use a Braille

Display Screen Reading Program when using this

Command.

INSTALL C#/L#          When using the TIEMAN CombiBraille Braille

Display connected to a Parallel Port.  Replace

the "#" after the Letter "C" with "2", "4" or

"8" to designate the Braille width as 20, 40

or 80 Characters.  Replace "#" after the

Letter "L" with the Parallel Port (LPT) used

by this device.  When using LPT1, the "/L1" is

not required, as LPT1 is assumed.

INSTALL H#/B#          When using a HANDY TECH Serial Braille

Display.  Replace the first "#" with the

Serial Port used by this device.  When using

COM1, the "1" is not required because the

Install Program will automatically assume that

COM1 is being used.  Replace the "#" following

the Letter "B" with 2, 4, or 8 to specify a

20, 40, or 80 Character Display.

INSTALL K#/B#          When using a KTS BRAILLOTERM Serial Braille

Display.  Replace the first "#" with the

Serial Port used by this device.  When using

COM1, the "1" is not required because the

Install Program will automatically assume that

COM1 is being used.  Replace the "#" following

the Letter "B" with 2, 4, or 8 to specify a

20, 40, or 80 Character Display.

INSTALL KB/L#         When using the TIEMAN KEYBRAILLE Parallel

Braille Display or TIEMAN MASTERBRAILLE

Portable PC.  Replace "#" with the Parallel Port

(LPT) used by this device.  When using

LPT1, the "/L1" is not required because

the Install Program will automatically assume

that LPT1 is being used.

INSTALL N#             When using a NAVIGATOR Serial Braille Display.

Replace "#" with the Serial Port used by this

device.  When using COM1, the "1" is not

required because the Install Program will

automatically assume that COM1 is being used.

INSTALL P#/B#          When using a POWERBRAILLE Serial Braille

Display.  Replace the first "#" with the

Serial Port used by this device.  When using

COM1, the "1" is not required because the

Install Program will automatically assume that

COM1 is being used.  Replace the "#" following

the Letter "B" with 2, 4, 6, 8, or 9 to

specify a 20, 40, 65, 80, or 81 Character Display.

INSTALL T#/B#          When using a TIEMAN Serial Braille Display or

TIEMAN PICO Integrated System.  Replace the

first "#" with the Serial Port used by this

device.  When using COM1, the "1" is not

required because the Install Program will

automatically assume that COM1 is being used.

Replace the "#" following the Letter "B" with

2, 4, or 8 to specify a 20, 40, or 80

Character Display.

**INSTALLING FOR BRAILLE AND SPEECH

When using a Braille Display and a Speech Synthesizer, type the

correct Install Command from the available Options explained in the

section concerning "Installing for a Speech Synthesizer".  Follow

the "#" in the Install Command with a single SPACE.

Now, type the correct Braille Display Option, as listed in the

previous section of this document.  You can even use the "B" Braille

Option to access a Menu of supported Braille Displays.  Although this

Menu will not appear in Braille until you have selected the correct

Braille Display and location for this device, the Menu will be spoken

through your Synthesizer.

**SAMPLE INSTALLATION COMMANDS

INSTALL 0

This will look for a DECTALK PC if the DECTALK Drivers are loaded.

The same is true if the Drivers are loaded for the SOUNDING BOARD.

It will also look for other Internal Synthesizers such as the

KEYNOTE PC, or DOUBLETALK PC.

INSTALL 1

This will look for a Serial Synthesizer connected to Serial Port (COM1)

INSTALL 2

This will look for a Serial Synthesizer connected to Serial Port (COM2)

INSTALL 1 B      Looks for a Serial Synthesizer on COM1, and

accesses a Menu of supported Braille Displays.

INSTALL 0 A34    Looks for an Internal Synthesizer in combination

with the ALVA 3-40 connected to LPT1.

INSTALL 2 A34/L2       Looks for a Serial Synthesizer connected to

COM2 in combination with the ALVA 3-40

connected to LPT2.

**TONE PROMPTS WHEN INSTALLING FOR TEXT ASSIST

As previously explained, 2 Disks are required when installing

WINDOW BRIDGE.

When installing for TEXT ASSIST, since there is not any verbal feedback

during the Installation process, you will hear a set of tones that

raise in Tone when it is time to insert Disk #2.  Likewise, there

will be a series of tones that drop in Tone to indicate that Disk #1

must be inserted when Install is finished with Disk #2.

Finally, a series of rapid Tones will be heard to indicate that

the Installation process for TEXT ASSIST is complete.

**THE INSTALL MENU

Once you have entered the appropriate INSTALL Command as outlined

in the previous section, press ENTER to begin the Install Program.

If you have specified that a Synthesizer is being used, you will

hear a series of musical tones that gradually raise in tone.

This series of tones will be heard as the Install Program looks

for your Synthesizer.  You may hear some characters being spoken

as the test proceeds.  These characters are codes that the Install

Program is sending out to confirm what specific synthesizer is

being used.

The exception  to this is when installing for the TEXT ASSIST Program.

When the Installation program has confirmed what adaptive

technology is being used, a Menu will appear on the screen.  This

Menu will allow a Full or Partial installation of the program to be

selected.  All Prompts and Messages will be sent to your

Synthesizer or Braille Display.

If you have used the "G#" Installation Option when using a Generic

Synthesizer, you will be presented with a vertical list of the

currently supported Generic Synthesizers.  Use the UP and DOWN

ARROWS to move to the correct synthesizer.  Press ENTER to access

the Main Installation Menu.

The available selections in the Installation Menu can be reviewed

with speech or braille output by navigating up or down the screen

with the UP and DOWN ARROW keys.  Pressing the SPACEBAR will read

Line 25 on the Installation Menu Screen.  This line contains the

Version of the program and the name of the recognized Speech

Synthesizer and Braille Display.  If the Menu shows that the wrong

Synthesizer or Braille Display has been recognized, press the

ESCAPE key to leave the Installation Menu.  Reset your system, and

execute the appropriate "Install" Command again.

It is strongly recommended that a FULL INSTALLATION be selected

when installing the program or a new revision of the program.

Partial Installations are used to re-install a portion of the

product without having to install all of the available files.

To read the current Menu Selection, or to repeat a prompt during

the installation process, press the LEFT or RIGHT ARROW.

Pressing the UP or DOWN ARROWS will move to a new selection.

Once you have reached the bottom of the list of choices, pressing

the DOWN ARROW again will wrap to the top of the Menu.  Pressing UP

ARROW at the top of the Menu will wrap to the bottom choice in the

list.

Each of the Menu choices begins with a different letter.  This

letter is the first letter of the selection.  The currently

highlighted command begins with the letter "F".  It is selected if

you wish to conduct a "FULL INSTALLATION".  To move directly to

another Menu Option; press the letter corresponding to that choice.

Press ENTER to begin, or ESCAPE to exit.

**INSTALLATION ERRORS

1. An Error Tone was heard a few seconds after pressing ENTER.

You did not log to the floppy Drive before pressing ENTER.

2. A quick series of tones that raise, then lower in Pitch is

heard a few seconds after pressing ENTER.

Your Serial Synthesizer or Braille Display is not connected to the

specified Serial Port, or is not turned, ON.

3. The musical tone list of Synthesizers is played 3 times,

followed by a double tone.

Your Synthesizer was not detected.  Press ENTER.

Verify that your synthesizer has been prepared properly, as was

discussed earlier in this document.  Specifically, be sure that

your Serial Synthesizer was connected to the Serial Port specified,

and that the synthesizer is turned ON.  When using a Synthesizer

that requires Speech Drivers to be loaded first, be sure that they

are loaded.  Also be sure that you were not trying to run the

Install Program while another Braille Display, Screen Review, or

Large Print Program was active.  These are the most common errors.

4. After about 15 seconds, 4 low tones are heard.

You just typed "Install" and did not specify a Braille Display, or

a location for a Synthesizer.  Read the section on using the

Install Command again.

5. The INSTALL MENU appears, but was not spoken or appear in

braille.

This rare condition will occur if the INSTALL program has

identified a modem, or other active Serial device which is behaving

similar to a valid speech synthesizer.  This can be verified by

reading the bottom line of the screen; where the identified

synthesizer is listed on the Status Line.  To correct this

situation; turn the external Serial device; OFF, or remove the

Internal Modem from the machine before running the INSTALL program.

6. ERROR READING SIGNATURE.

This message indicates one of two things: this is not a MASTER

DISK, or the computer has a non-register level compatible Disk

Drive Controller.  Contact Syntha-voice Computers when you are

at your PC for further instructions.