Braille 'n Speak Disk Accessory Manual

Braille 'n Speak

Disk Accessory

OWNERS MANUAL

by Deane Blazie

Release Date: January 1991

Blazie Engineering

Blazie Engineering

109 E. Jarrettsville Road

Forest Hill, Maryland  21050

(410) 893-9333

fax (410) 836-5040�

Braille 'n Speak

Disk Accessory

Table of Contents

1.0 Introduction	3

2.0 Using the Disk Accessory with the Braille 'n Speak.	3

2.1  Power Switch, Cables and Charging.	3

2.2 Turning On The Power	4

2.3  Braille 'n Speak Disk Commands	5

2.3.1  Loading A File into The Braille 'n Speak.	5

2.3.2  Saving A Braille 'n Speak File Onto Disk	6

2.3.3  Getting A Directory Of The Disk	7

2.3.4  Deleting Files From Disk	7

2.3.5  Formatting A Disk	8

2.3.5  Creating Subdirectories On A Disk	8

2.3.6  Deleting A Subdirectory	9

2.3.7  Reading Very Large Files	9

2.3.8 Examples	9

2.4  Things To Be Aware Of	10

3.0  Using The Serial Interface	10

TABLE 3.1 Disk Serial Errors	12

4.0  Problems, Questions and Suggestions	12

.c.1.0 Introduction

The Braille 'n Speak disk accessory is a small disk storage device

that lets you save files from Braille 'n Speak onto standard 3.5 inch

disks.  When used with Braille 'n Speak you can store files, load files

back into the Braille 'n Speak and get directory listings of what's on

your disks.  Each disk can hold up to 1.4 million characters of

information.  The disks are MSDOS compatible which means that you

can take them to your PC compatible and read them.  You can transfer

a file from your PC to the disk and then read the file into your

Braille 'n Speak.

The Disk Accessory is battery powered so you can use it anywhere

you use the Braille 'n Speak, and its only a little larger than the

Braille 'n Speak.  The rechargeable nickel cadnium batteries last

from 5 to 10 hours before they need recharging.  The disk accessory

uses the same charger as the Braille 'n Speak.  This is a 9 volt DC,

positive tip 200 ma charger.  The disk plugs into the second port on

your Braille 'n Speak.  The drive is simple to use and can read and

write either normal density 720Kbyte disks or the High Density

1.44Mbyte disks.

With the optional serial cable, the disk accessory can also be used

with other devices.  The commands are similar to the Braille 'n

Speak commands.  Other devices include, PC's, Laptops, Personal

Touch, Pocket Braille and virtually any other RS-232 device.

This manual describes how to use the disk drive with the Braille 'n

Speak and also with the serial adapter cable.

.c.2.0 Using the Disk Accessory with the Braille 'n Speak.

.c2.2.1  Power Switch, Cables and Charging.

The disk accessory comes with a cable to connect to a Braille 'n

Speak.  This cable plugs into the 6 pin port on the left side of the

Braille 'n Speak.  This port is immediately next to the normal serial

port and toward the front of the Braille 'n Speak.  The connector is

keyed so that it will only go in one way, that is with the small tab

pointing up.  Don't force the connector into the Braille 'n Speak as

this might bend one of the pins.  It should go in very easily.

The disk drive is a small unit measuring 8 1/2 inches deep by 5 1/2

inches wide by 2 inches high.  The front panel has a slot where the

disk is placed.  The disk will only go in the correct way.  Do not

force a disk in the drive.  The disk will slide in and snap into the

disk drive.  The disk can be removed from the drive by pressing on

the button just below the disk slot and to the right of center on the

front of the disk drive.

On the back panel of the drive unit is the power on/off switch, the

charger jack and the connector for the drive cable.  Facing the rear

panel the drive power switch is near the lower right.  It is a rocker

switch and the disk is powered on by pressing on the right edge of

the switch.

Note, the drive will not come on unless a cable is plugged into the

connector on the back of the drive unit.  This is a safety feature so

that if you are carrying the drive in a briefcase or pack, it will not

accidently come on if the switch is pressed.

The charger jack is just to the left of the power switch.  It is not

necessary to plug in the data cable to charge the drive, just to

operate it.  The drive will fully charge in about 8 hours.  There is no

battery low indicator on the disk drive.  So it is important to keep

track of usage and charge the batteries accordingly.  You can also run

the disk unit while the charger is plugged in but remember,  the disk

motor draws heavily on the battery and there must be some charge in

the battery in order to operate the disk.

.c2.2.2 Turning On The Power

After the disk unit is cabled to the Braille 'n Speak turn on the

power switch.  The disk unit should respond with a door bell sound or

"bing bong".  This is the only indication that the unit is operational.

If you don't hear the bing bong turn off the power, check all the

connections and try again.  If the batteries are dead, you will have to

charge them first.

The disk has a built in reminder that the power is on.  If the disk is

not used for about 5 minutes, it will sound a "bong bing" to let you

know it is on.  This helps prevent leaving the unit on and running

down the batteries.

New with the December revision.

Battery Low Indicator:  The disk drive (December 1990 revision and

later) has a battery low detection circuit built in.  The battery

voltage is monitored and if the voltage goes below a certain

threshold (4.65 volts) the disk drive will beep continuously.  At this

time the disk drive should be charged.  If you continue to use it

while the battery is low you may experience problems and could lose

information.

.c2.2.3  Braille 'n Speak Disk Commands

With the disk drive powered on and operational, you can begin to use

it.   You must have a Braille 'n Speak with a July 1, 1990 or later

revision.  To determine your revision date, enter the P chord R

command on the Braille 'n Speak.

All disk commands begin with S chord on the Braille 'n Speak.

The commands are:

S chord L loads a file from the disk into Braille 'n Speak.

S chord S saves the current file onto disk.

S chord T saves the current file on disk as if printed.

S chord D lists a directory of the disk.

S chord R resumes reading a large file.

S chord K deletes a file from disk.

S chord F formats a disk.

S chord M makes a subdirectory on the disk.

S chord X deletes a subdirectory on the disk.

Before describing the commands it is important to know a bit of the

philosophy of operation of the disk drive.  The disk is simply a

storage device for Braille 'n Speak files.  You can transfer files from

the disk to a file within Braille 'n Speak or you can Send a file in

Braille 'n Speak to the disk and give it a name.  You cannot look at

files or change them while they are on the disk.  You must first read

them into the Braille 'n Speak and then operate on them.

.c3.2.3.1  Loading A File into The Braille 'n Speak.

In order to load a file into the Braille 'n Speak, you must first make

room in the Braille 'n Speak to put the file.  You may want to create a

new file on the Braille 'n Speak to put the information or you may

want to read the information form the disk into an existing file at

the end of the data.  When you enter the S chord L command the data

will go into whatever file is presently open.  You must be in the file

ready to work on it.

When you enter S chord Braille 'n Speak will say Storage.  You can

then enter the storage command you wish to perform.  Entering L for

load will cause Braille 'n Speak to say "ENTER FILE NAME".  Now enter

the name of the file you wish to load into the Braille 'n Speak and

press E chord.  You should now hear the disk drive working and a

light on the front of the drive will activate.  The disk accessory will

now read the directory on the disk and try to locate the file name

you entered.  If it finds the file on disk, it will append the data to

whatever file is currently open on the Braille 'n Speak.  This allows

you to combine files together.  It also keeps you from writing over

the top of a file already in the Braille 'n Speak.

If the file doesn't exist on the disk Braille 'n Speak will speak the

message "FILE DOESN'T EXIST".  Simply try another file name or

check the directory of the disk with the S chord D command.

If the file does exist on the disk it will be sent into the memory of

the Braille 'n Speak.  However, if there isn't enough room in the

Braille 'n Speak file which is currently open, the Braille 'n Speak

will instruct the disk to stop reading and will say "FILE IS FULL".  At

this point the disk will wait for further instructions (See S chord R).

The Braille 'n Speak will have part of the file in memory.  You can

read the rest of the file by first making enough room in the Braille 'n

Speak and then telling the disk to resume (S chord R).

.c3.2.3.2  Saving A Braille 'n Speak File Onto Disk

To save a file on disk just open the file to be saved and enter the

command S chord S.  The Braille 'n Speak will ask "ENTER FILE NAME".

It is asking for the name you wish to assign to the file when it is

stored onto the disk.  This name doesn't have to be the same as the

name of the Braille 'n Speak file.  In fact, if the name is longer than

12 characters it can't be used since the disk drive can only use file

names which are 8 characters period 3 characters.  After entering

the file name enter E chord.  The disk should begin spinning and the

file will be sent from the Braille 'n Speak and stored on the disk.

The file will be stored just as it exists in the Braille 'n Speak.  If it

is a Grade 2 Braille file it will be transferred to the disk as a Grade

2 file.  If it is a computer Braille file it will appear on disk as a

computer Braille file.  If you are storing files on disk to be later re-

read back into the Braille 'n Speak then this is what you want.

If you are storing a Grade 2 file and are going to read it into a PC and

want it translated into text you should use the S chord T command.

This will transmit the file to the disk after formatting the file just

as if you had printed it with the T chord Z command.  This command

is handy but remember if you read the file back into the Braille 'n

Speak you will have a text file, not a Grade 2 file.

.c3.2.3.3  Getting A Directory Of The Disk

Braille 'n Speak can read the directory of the disk by executing the S

chord D command.  You will be asked for "FILE NAME".  It is really

asking for the file mask or directory mask just as in the MSDOS DIR

command.  For example, if you enter a file name of *.brl you would

get a directory of all files on the disk with ".brl" as the file

extension.  Simply entering E chord with no file name will give all

files on disk.

Where does the directory go?  When you ask for a directory Braille 'n

Speak automatically closes the file you are in and puts you into the

clipboard file.  It then clears the clipboard and stores the disk

directory into the clipboard, putting the cursor on the first file

name.  You can now read through the directory in clipboard and find

the files you want.  You can use all the power of the Braille 'n Speak

to read the directory.  F chord to find a file, skipping lines, etc.

When you are done reading the directory, you can simply enter Z

chord and the clipboard will be closed and you will be back into your

original file.  The directory will remain in clipboard until you do

some operation which clears it.

Notice that at the end of the file directory the ROM revision date of

the disk accessory is given.  This may be helpful if you experience

problems with the disk drive and have to contact Blazie Engineering.

.c3.2.3.4  Deleting Files From Disk

The command S chord K will delete a file from the disk.  Braille 'n

Speak will ask for the file name and wait for you to enter and E

chord.  You may also use MSDOS wild card characters (* and ?) in file

names to delete but Braille 'n Speak will not let you do a delete all

using *.*.  If a file doesn't exist or if the disk is protected, Braille 'n

Speak will issue an error message and you can start over.

.c3.2.3.5  Formatting A Disk

When you purchase 3 1/2 inch disks they normally come to you in an

unusable state.  That is, they must be "formatted".  This formatting

is simply a process of writing markers on the disk and storing a

blank directory.  If you don't format a disk and you try to use it you

will get an error of some kind.

To format a disk simply insert a blank (or used) disk in the drive and

enter the command S chord F.  Braille 'n Speak will say "FORMAT

DISK, ARE YOU SURE, ENTER Y OR N".  Since formatting will erase all

of the information on the disk Braille 'n Speaks asks if you are sure.

Enter Y if you want to continue or N if you want to abort the

formatting.  Formatting a disk takes several minutes and disk gives

a beep every time it formats a track.  There are 80 tracks on a disk.

After formatting a disk you can use it immediately either in the

Braille 'n Speak disk accessory or your PC compatible computer.

.c3.2.3.5  Creating Subdirectories On A Disk

You can create directories within directories with the disk

accessory.  We call these subdirectories.  Please refer to an MSDOS

manual for a complete description of subdirectories and paths.

Braille 'n Speak uses the command S chord M to 'make' a subdirectory

on a disk.  You can use a standard MSDOS path in the directory name.

When you enter the S chord M command Braille' n Speak will ask for

the "FILE NAME".  It is really asking for a subdirectory name.  Just

enter the name of the subdirectory you wish to create and press E

chord.  Note that to if you want to enter an MSDOS path you must use

the backslash character.  To get this character as part of the file

name you must use the U chord then 1 2 5 6 character.  This is the

uppercase OU sign in Braille.  Braille 'n Speak has been modified to

accept the U chord and OU sign.

.c3.2.3.6  Deleting A Subdirectory

Deleting a subdirectory is just like deleting a file except you use the

S chord X command.  Braille 'n Speak will ask for the "FILE NAME" to

delete.  Enter the name of the empty subdirectory and it will be

deleted.  If the directory has files in it you will be given an error

message that it cannot be deleted.  Remove the files and try again.

.c3.2.3.7  Reading Very Large Files

Sometimes you may have a file that you want to read into the Braille

'n Speak but it will not fit into the available memory.  You may still

be able to read it piece by piece with the S chord R command.  All

you have to do is open a file and enter the S chord L command to read

in the file.  When the file you are reading into is nearly filled the

Braille 'n Speak will say "FILE IS FULL".  The disk drive will stop and

you are able to use the data read in so far.  When you have made more

room (by deleting information or adding pages) you can resume

reading by entering S chord R to get more data into the Braille 'n

Speak.  You can read very long files this way even into a one page

Braille 'n Speak file.  Just keep deleting text after reading and then

enter the S chord R command to get another piece.

.c3.2.3.8 Examples

Storing the HELP file on disk.  First open the HELP file in the Braille

'n Speak.  Now, with a formatted disk in the disk drive enter:

S-chord S HELP E-chord

There are no spaces in the actual command, just in this text to make

it more readable.  Braille 'n Speak will send the file to the disk and

say OK.

Now read the directory of the disk and verify that the HELP file got

saves.  Enter:

S-chord D E-chord.  Braille 'n Speak will load the directory into

clipboard and say OK.  Now you can read the clipboard and you should

find the help file you stored.

Loading the help file back into another Braille 'n Speak file.  First

create a file called HELP2 or any name. and open that file.

Enter S-chord L HELP E-chord.  The disk should run and Braille 'n

Speak should say OK.  Now read this file and you will find it to be

identical to the HELP file.

You can practice on the disk that comes with the disk accessory.

This disk contains various manuals on the Braille 'n Speak and the

Disk Accessory.  You can print these files in print or Braille at your

convenience.

.c2.2.4  Things To Be Aware Of

You may notice that if you store the HELP file, one page long onto the

disk it will store just fine.  However if you open a one page file and

try to load the HELP file back into memory Braille 'n Speak will load

all except about 250 characters and it will say "FILE IS FULL".  This

is because of the way handshaking is done on the Braille 'n Speak.

Whenever a file is within 256 characters of being full Braille 'n

Speak will tell the sending device to stop sending.  It does this at

256 characters because some computers cannot stop immediately

and so we leave some room.  This causes the minor irritation when

dealing with files which are nearly full.  To get around it, just make

the file one extra page long and then after reading the file in delete

the extra page.

You may also work with some floppy disks that have such a long

directory that it will not all fit into the clipboard.  Braille 'n Speak

will say file is full.  You can use the S chord R command to read the

rest of the directory after you have made room.

.c.3.0  Using The Serial Interface

You can control the disk drive accessory from any serial device by

using the optional serial cable available from Blazie Engineering.

This cable has a 15 pin connector on one end to plug into the disk

accessory and a 25 pin Female connector on the other end which will

plug directly into a serial port on a PC.  This cable makes the disk

drive look like a DCE to the connecting device.  In other words it

looks like a modem.

The disk drive accessory is set to operate at 9600 baud, 8 data bits,

no parity and one stop bit.  When you power on the disk drive you will

get a command prompt character '@'.  To use the disk from a serial

device simply replace the S-chord in the commands above with a

control-E character (ASCII decimal 5 ENQ character).  and end the

command with a carriage return instead of an E-chord.  For example

to get a directory of all files on the disk send "control-ED return".

This is only 3 characters, one for control E one for the letter D and a

carriage return.  Getting a directory of all files ending in .BRL would

require the command "control-ED*.BRL return".  Whenever a command

is completed successfully a control Z character is sent from the

disk drive.  If an error occurs an error character is sent.  This single

character can be interpreted TABLE 3.1 to determine the error.

IMPORTANT:  When storing data to the disk from the serial

interface use the command "control-ESFNAME return".  This command

tells the disk to create the file "FNAME" on the disk and to wait for

data to come in the serial port.  As data is received it is put onto the

disk sector by sector.  The disk will continue waiting for data until

it receives a "control-Z" character.  This is the end of file character

and tells the disk drive to write the last block onto the disk and to

update the directory information.

The disk drive uses XON/XOFF handshake protocol to control the flow

of information to and from the disk drive.

.c2.TABLE 3.1 Disk Serial Errors

Command not recognized 	?

File not found	#

Disk write protected	!

Format, CRC or sector failure	&

Insufficient disk space	%

Delete all (*.*) is illegal	"

No disk in drive	$

SubDirectory already exists	+

SubDirectory does not exist	-

Directory not empty	/

.c.4.0  Problems, Questions and Suggestions

If you have any problems, questions or suggestions on any of our

products, please call or write to us.  We are very interested in

hearing from you.  A full service maintenance contract is available

from Blazie Engineering for the disk accessory.  This contract

includes sending an exchange unit to you in the event your disk

accessory has a problem.

Blazie Engineering

109 E. Jarrettsville Road

Forest Hill, Maryland  21050

(410) 893-9333

fax (410) 836-5040�