N  R  D

A Control Program / Database for Japan Radio NRD-525 and NRD-535 Radios

by Tom Whiteside

NRD is a control program / database for NRD525 and NRD535 receivers and

requires an IBM or compatible computer.  This program is distributed as

"Freeware" as a token repayment for the valuable assistance received over the

years from other DXers in the form of articles and log reports.  NRD is

designed to be useful for a wide range of shortwave interests from casual

listeners to fanatical broadcast and utility DXers.  The program is intended

to be easy to operate for new users and also provide more sophisticated

features that will take some practice.  I recommend printing this document and

trying the various commands with the radio.  A rather complete demo mode

simulates the program's behavior without a radio and is useful for evaluating

radio purchase decisions and as a stand alone radio log.

PHILOSOPHY:

I got tired of using paper for logging many moons ago.  With paper logs, its

tough to go back and find where radio xyz is, etc.  Much continues to be

written on ways to organize paper records -- no thanks!!!  The first step was

logging on paper, then entering the data in a Spreadsheet.  Now, things could

be found but this is tedious!  This program eliminates the middle person!  The

next step was achieving the right balance between controlling the radio

manually and from the keyboard.  Basically, you can do either with this

program.  Sloth is the watch-word throughout and every attempt is made to

minimize effort.  For example, to log a station, hit the "L" key and the

program does all the work to automatically grab the frequency, date, time (and

the rest of the receiver state) leaving you to enter only the ID, Location and

a comment if you wish.  Keys like "+" and "-" can be used to tune in

programmable increments (like 5 kHz) to speed bandscanning.  The program can

automatically track radio frequency changes and display database records for

this frequency.  The "T" key will tune a selected database entry and load the

radio with the appropriate parameters for that entry. (Like mode, bandwidth,

etc.  Other keys control other radio parameters such as the bandwidth and mode.

Different databases can be seleted, sorted, searched, etc.

COMMAND CONVENTIONS:

In the documentation, commands are described in quotes but are initiated by

merely typing the letter without the quotes.  For example, the "T" command

is activated by hitting "T" or "t". Commands do NOT require hitting the "Enter"

key.  Generally, the "Enter" key is only needed when entering data where the

length of the data is not known (like a city).

RUNNING FOR THE FIRST TIME:

To run the program, type: NRD and hit the Enter key.

Note:  If the program is to be used with a monochrome display, delete the file

entitled @MONITOR.DAT prior to running NRD and do as directed.

The first time this program is run, it creates a file entitled:  CONFIG.DAT

which is used to store data unique to your situation.  You will first be

prompted to enter the COM port for use with the radio.  (Ports 1 and 2 are

supported.) If you specify "0" for the COM port, the program will run in demo

mode.  In demo mode, you can use the database and the radio commands are be

simulated.  (I use demo mode when traveling and it can be used as a step up

from paper logs by folks without JRC radios.  You will next be

asked to enter the type radio you have.  Currently supported receivers are the

525 and 535 (Enter just the radio number:  e.g.  535).  If you selected 525,

you will be asked if you have a KIWA MAP Unit.  If you don't have one, simply

type "n" for "no".  Saying "y" for "yes" will enable a "K" for "KIWA" command

described later.  (This feature is only available with the NRD525.) You will

next be prompted to enter the current time GMT.  This is important.  When

logging stations, the program will use this information to convert from your

computer's time to GMT.  You will be prompted to enter a printer control string

used to place the printer in compressed mode.  Check your printer manual for

the proper value to enter.  Look up the appropriate sequence and enter the

ASCII values with intervening spaces when prompted.  For example, for an EPSON

LQ-500, the required sequence is "escape" "g".  For this, enter:  27 103 and

hit the Enter key.  Many printers use 15.  Similarly, you will be asked to

enter the ASCII values to restore the printer to normal operation.  To restore

an EPSON LQ-500 to 10 pitch printing, enter 27 80.

Finally, you will be asked if you wish to control a second radio.  Enter "y"

if you have a second radio (NRD525 or NRD535) you wish to control as well.  If

you indicate a second radio, you will be prompted to enter the COM port and

radio type as done above.

If any of these values need changing later, simply delete the CONFIG.DAT file

and re-enter the appropriate data.  (See also the "Configuration" option for

the "Overall" command described later to change these settings without leaving

the program.)

After the CONFIG.DAT file is completed, the program main prompt will appear.

FIRST TIME USERS:

Before stations can be logged or other database commands will work, a database

must be created.  If you are a new user to NRD, you will probably want to

"import" the "sample" database to get the feel of the program.  "Importing" is

described later but to get started, Hit "J" and you should see the "Journal"

prompt.  Hit "I" for "Import"; "A" for "ASCII".  You will then be prompted to

Enter the name of the database to import.  Type:  "SAMPLE" and hit the ENTER

key.  The program will next ask you for the name to save this under.  Type

some name like:  "JUNK" and hit ENTER.  Now type "Q" for "Quit" to leave

"Journal" The program will now ask you for a database to view - Type in the

number corresponding to the database you just imported (should be "1") and hit

ENTER.

PROGRAM OVERVIEW

Now all that was a lot of work that you won't have to do the next time.  Now to

describe the main display structure.  The screen is divided into 4 sections:

�������������������������������������������������������������������Ŀ

�                      Command Prompt Area                          �

�������������������������������������������������������������������Ĵ

�                                                                   �

�             Radio Status Area (Frequency, AGC, etc)               �

�                                                                   �

�������������������������������������������������������������������Ĵ

�                                                                   �

�                                                                   �

�                                                                   �

�                                                                   �

�                     Active Database Area                          �

�                                                                   �

�                                                                   �

�                                                                   �

�������������������������������������������������������������������Ĵ

�                      Program Status Area                          �

���������������������������������������������������������������������

The Command Prompt Area has a list of active commands or other prompts used in

the program.  The commands are described in the COMMANDS section.  The Radio

Status Area contains information about the Radio such as its frequency, mode,

bandwidth setting, etc.  See the section on Controlling the Radio for more on

this.  The Database Area is a window to the list of stations logged in the

active database.  See the section on Controlling the Database for how to use

the database to tune the radio, add more entries, etc.  Finally, the Program

Status Area displays the active database name, the inactive database (if any)

when you switch to a new database, the previous one used becomes the inactive

database.  This has some advantages like single keystroke toggling back and

forth and is described later.) Other items in the Program Status Area are the

"Scan Threshold" and "Mode Status" (described later and present ONLY for 535

users) and the UTC time and date.

COMMANDS:

These will be described in three sections:  CONTROLLING THE RADIO, GRAPHICS

COMMANDS and DATABASE COMMANDS.  A prompt line at the top of the screen serves

as a reminder of commands available at a particular level in the program.  A

typical prompt line might look like:

NRDx.y: L(og, C(onfirm, S(ort, E(dit, T(une, P(age, H(elp, Q(uit [/]

The "x.y" after the program name indicates the program version number.  The

CAPITALIZED letter to the left of the "(" indicates the letter to type to

initiate a given command.  Commands are not case sensitive.  For example, "L"

or "l" to perform a "Log" command (described later).  The "[/]" indicates that

other commands are available at this level and these can be seen by hitting the

"/" key.  All commands at a level are active even though they may not be

displayed on the prompt line.  Some commands require depressing the CTRL key

and selecting a letter.  These are indicated on the prompt line as CTL-x where

x is the letter for the command.  For example, "CTL-uN(mark line" indicates

that hitting CTRL and "N" keys together initiates the "uNmark line" command.

Likewise, some commands require the ALT key and are shown on the prompt line as

ALT-x.

CONTROLLING THE RADIO:

This program allows you to control the radio either manually or with the

computer keyboard.  To do this, mostly the radio is kept "unlocked" by the

program.  When you manually change the radio, the new radio settings will be

automatically sent to the computer and displayed in the Radio Status area.

Changes in the Program Status Area are always highlighted in RED.  Try changing

the mode, bandwidth, AGC and frequency settings from the radio and observe the

effects on the program display.

CONTROLLING THE RADIO COMMANDS:

"+" : The meaning of this command is a function of the increment mode and

there are four basic modes.  The 535 supports all modes and the 525

supports the "F" and "T" modes.  Setting these modes is done with the

"Increment mode" command described later.  It is recommended that first

time users stick to the default "Frequency mode" described below until

familiar with the program.  The modes are:

"F":  Frequency mode:  In this mode, hitting the "+" key increments

the frequency by 5khz. (See the "Overview" command to change

the increment to some other value.)  This is the most common

usage and the initial program default mode.  If the mode is USB

or LSB and the increment is 5khz, the radio is momentarily

detuned by a khz to aid in detecting weak stations with the

carrier heterodyne.  Bandscanning with "+" is VERY fast!

"N":  Next above threshold:  This is like the "F" command but keeps

bumping the frequency in 5khz increments (See the "Overview

command to change the increment amount) until an above

threshold signal is found. (see the "CTL-T" command for

adjusting the threshold; A threshold setting of 0db initiates a

special case where the radio's squelch status is used rather

than the s-meter for determining above threshold signals.)

The radio is set for Narrow bandwidth and CW mode during the

search to minimize fooling the S-Meter with co-channel energy.

After an above threshold signal is found, the radio's mode

settings are restored to the values before activating the

command and the radio is tuned to this new frequency. The radio

front panel remains locked during the search.

"M":  switch to next Marked entry that is above threshold:  See the

documentation on Marking database entries ("M", "CTL-M", "U",

and "CTL-L").  The next sequential marked entry past the

current cursor position is tuned and checked for being above

threshold.  If the signal is below threshold, the next marked

frequency is selected and so forth.  When the end of the list

is reached, the search begins again at the top.  (See CTL-T

for adjusting the threshold; A threshold setting of 0db

initiates a special case where the radio's squelch status is

used rather than the s-meter for determining above threshold

signals.) The radio front panel remains locked during the

search.

"S":  Scan the marked entries for an above threshold signal:  This is

similar to the above "M" mode except that scanning will be

automatically resumed if the signal drops below threshold.  An

example usage for this is listening to duplex marine telephone

signals.  Hitting "+" or "-" forces scanning to resume.  Any

other key halts scanning.  The radio front panel remains locked

during the scanning. (See CTL-T for adjusting the threshold; A

threshold setting of 0db initiates a special case where the

radio's squelch status is used rather than the s-meter for

determining above threshold signals.)

"T":  Continue scanning through marked entries until a key is

pressed: This is similar to the "M" mode except your ear

determines an above threshold or otherwise interesting signal.

A programmable time delay (see the CTL-T command) determines

the length of time the program will pause on each station.

Note: if the database is sorted by frequency and Automatic Frequency

Tracking is enabled, the highlighted database entry will

automatically follow the frequency.  Automatic frequency tracking

defaults to ON and can be overridden with the Overall command.

Note: "=" is an alias for "+" for keyboards for users who prefer to

not reach for the numeric keypad area.

"-" : this command is the same as the "+" command but acts as a decrement

rather than an increment.

"<" : decrements the receiver "mode".  e.g. LSB to USB.  "," works as well so

no shifting is needed.  The sequence order matches the radio manual

control.

">" : increments the receiver "mode".  e.g. USB to LSB.  "." works as well so

no shifting is needed.

"[" : decrements the bandwidth.

"]" : increments the bandwidth.

"F1" : set RTTY mode.

"F2" : set CW mode.

"F3" : set USB mode.

"F4" : set LSB mode.

"F5" : set AM mode.

"F6" : set FM mode.

"F7" : set FAX mode.

"F8" : set ECSS-USB mode.  (NRD535D Only)

"F9" : set ECSS-LSB mode.  (NRD535D Only)

"C-F1" : set bandwidth to Narrow.

"C-F2" : set bandwidth to Intermediate.

"C-F3" : set bandwidth to Wide.

"C-F4" : set bandwidth to Auxillary.

"|" : changes the Bandwidth Control (BWC) setting. (NRD535D only)

"\" : changes the Passband Shift    (PBS) setting. (NRD535D only)

"ALT-A" : Agc toggle between FAST and SLOW modes

"CTL-N" : "Nap" - toggle power to the radio (NRD535D only)

"CTL-J: : "Jack" - toggle the timer relay (NRD535D only)

" " : (space) refresh the receiver display.  This is not generally necessary

since receiver changes are echoed back to the program automatically.

The exception is user channel changes which are not automatically

echoed by the receiver. This can also help recover from user errors

like having the radio off when the program is run.

"F": for Frequency set - This command is used for entering a frequency to

tune the radio to.  The program will set the radio frequency and

locate the nearest corresponding entry in the database (if sorted in

frequency order and Automatic Frequency Tracking is enabled).

Note:  Hitting ENTER without typing a frequency aborts the command.

"ALT-": decrements frequency by 0.05 Khz.  (Note - does not work with keypad

minus.)

"ALT+": increments frequency by 0.05 Khz.  (Note - does not work with keypad

plus.)

"R": for "s-meteR" - applies only for NRD535 users.  Toggles mode of

displaying the S-Meter reading in the receiver status.  Default is OFF

for PROM versions prior to "F" and ON for Version "F" and beyond.  The

Version "F" PROM eliminates all overhead and side effects associated

with using the S-Meter.  Earlier versions require briefly locking the

front panel and override PBS settings.

"I": for set Increment mode.  When this command is activated, you

will be prompted to select "F", "N", "M", "S" or "T" which selects the

mode for use with the "+" and "-" commands.  The current mode is

displayed in the Program Status Area of the display.  The program

default is the "F" mode.  The NRD525 S-Meter cannot be read and

therefore only the "F" and "T" modes are valid options.

"CTL-T": for set Threshold or Time delay.  Depending on the context, this is

used to set either the threshold dB setting or time delay associated

with the "+" and "-" commands.  The Up and Down arrow keys are used

to change this value after this command is selected.  Type any other

key when the desired value is reached.  This value is displayed in the

Program Status Area.  This command affects scanning threshold values for

NRD535 users in all but the "Timer scan mode".  In Timer mode for both

the NRD535 and the NRD525, the setting corresponds to the time delay

used for scanning.  A threshold setting of 0db initiates a special case

where the radio's squelch status is used rather than the s-meter for

determining above threshold signals.)

"K" : for "Kiwa" - If in LSB or USB, puts the radio in AM with WIDE bandwidth

(and detunes by 2 khz).  If in AM, puts radio on nearest 5 khz center

and puts in LSB or USB (depending on the offset) and sets the bandwidth

to INTER.  Used to toggle high fidelity listening for use with KIWA MAP

unit.  Unless you answer "yes" to has KIWA MAP unit in building the

CONFIG.DAT file, this command will not appear or work.  This feature is

not supported for the NRD535.

"ALT-F": for "Fidelity" -  Toggles mode and bandwidth between USB with

Intermediate Bandwidth and ECSS USB (AM for 525) with Wide Bandwidth or

LSB with Intermediate Bandwidth and ECSS LSB (AM for 525) with Wide

Bandwidth.  Used to select high fidelity listening conditions when

conditions permit.

"O" : for "Overall" - for the 525 or 535, this command allows the user to

change the frequency increment associated with the "+" and "-" commands

to some other value,  to modify the display option (described below)

and to modify a speed constant associated with computer-radio

interaction.  With the 535, the following receiver settings may be

changed with these commands:

"T" : for "Tune rate" (toggles tuning rate between 3 speeds)

"F": for "Filter bypass" (toggles receivers filter bypass)

"O": for "bfO offset" (enter value from -5000 to +5000Hz)

"B": for "ssB display shift" (toggles display shift)

Stats option:  This overall command allows clearing scan hit

statistics, changing the scan hit time plot sample interval, and

changing the default time that scan time plot displays begin with.

(See the eXchange command for more on scan statistical displays)

Inc options:  This Overall command is used to change the frequency

offset used with the "+" and "-" commands.  The default is 5khz.

Display options:  The active database can be displayed in one of three

options selected by the "D" command.  This prompt looks like:

Display entries: A(ll, H(ighlight active, O(nly active, Q(uit

The "A" for "All" option is the program default and displays all

entries for the active database.  "H" for "Highlight" is similar but

stations that are not currently active (as determined by the time

ranges for the log entries) are displayed in dark gray as opposed to

active stations which are a cyan color.  This is my personal favorite

mode as it makes it easy to see which stations are active or need to

be edited or confirmed to indicate their active status.  The "O" for

"Only" mode displays only stations currently active.  This mode is

intended for tuning stations from a database with minimum clutter but

is not recommended for bandscanning.  Caveats on "Only" mode are that

performance is degraded slightly due to having to sift through more

data per screen displayed.  The various "Mark" and unmark commands are

designed to affect only displayed entries to simplify the process of

tagging only active entries.  (See also the "Zulu" command.)

actiVe mode:  This overall command toggles between using the exact

times in a database log or times rounded to the nearest 1/2 hour to

determine a stations active status for display.  (See the display

option above).

Automatic frequency tracking: This option toggles this mode.  When

enabled (the default), all frequency changes made to the radio are

automatically tracked on the database screen.  This works well only

when the database is sorted by frequency.

Radio delay allowance:  This option allows changing the delay used by

the program in interacting with the radio.  Too low a value can result

in erratic operation -- especially with the 525.  NRD535 receivers

with version F PROMs or later should work fine with zero delay.  I

recommend leaving these values alone until you are fully familiar with

the operation of this program.

Measure scan statistics:  Turning this off prevents scan statistics

from being created and locks out the scan display pages for histograms

and scan time plots.  Turning this option off saves disk space and

the bother of seeing these screens if statistics are not useful to you.

There is no appreciable performance difference in scan speed with

statistics.

save Pbs/bwc values in db:  Selecting this option toggles various

conditions for saving the values for the Pass Band Filter and Bandwidth

Control at the beginning of the comment field automatically with the

"Log" or "Confirm" commands.  The choices, which toggle each time the

"P" key is pressed from this Overall screen, are (1) NEVER save these

values, (2) save them ONLY when in RTTY mode, (3) save them for all

radio modes EXCEPT AM or (4) ALWAYS save them.  Why save these?  The

values for the PBS and BWC will be asserted automatically to the radio

as part of the "Tune" command if they are found in this comment field.

The comment field will begin with B/P=xxxx/syyyy where xxxx = the BWC

value and syyyy = the PBS value (s = + or -).  Any user comments get

shifted to the right in the comment field and any characters in the

last 15 characters are lost.  These values will not be saved in the

comment field if both the BWC and PBS are at nominal settings.

Log/confirm tags entry:  Selecting this option toggles this mode.  When

the mode is "Yes", the tag field (*) is set when ever an entry is

logged or confirmed.  This is VERY useful for tracking changes to your

database.

Y axis - lines per screen:  This toggles the screen display between

three modes:

(1) 24 lines per screen (default)

(2) 43 lines per screen (for EGA) or 50 (for VGA)

(3) use screen mode in use prior to running the program.

The third mode is provided at the request of users that have a

favorite (hardware specific) screen mode (eg 37 lines per screen).

Options (1) and (2) are the only modes setable by Turbo Pascal.  Option

(3) puts responsibility on the user to select a workable mode - for

example, a 40 character wide display mode will produce wierd results.

Note: This command has no effect with CGA screens.

Detailed log status:  This toggles between the default single line

status display and a two line staus display.  The extra status line

displays the number of records, tagged entries, and marked entries in

the database.

Configuration:  This option rebuilds the user configuration data in the

same manner as when the program was first run.  This is useful for

updating Printer codes or time information.  In demo mode, radio types

can be changed to evaluate feature differences between the 525 and 535.

"Z" for "Zulu" - toggles between the two most recently used display modes.

See "Overview - Display" for a description of the display modes. I

use Highlight and Only Active display modes and this is faster than

using the Overall command.

"X" for "eXchange" - toggles between regular and statistics display modes.

In statistics display mode, only marked frequencies are displayed and

special display pages (horizontal pages 4 and 5) are enabled to view

scan statistics.  Page 4 shows a histogram of scan hits.  Page 5 shows

a time plot of scan hits.  Scan time histories look something like:

___-__  where the "_" indicates above no threshold signal was received

in the sample interval and "-" indicates a hit.  The sample interval

and the display start times are set with the Overall command.  A total

of up to 128 intervals are stored.  These pages are accessed by the

"P" command which works identically with the regular display mode.  The

"X" command works in conjunction with the "Z" command so, for example,

only active marked stations can be viewed with the right combination of

these commands.  (See also the Overall command for altering display and

statistics.)  Statistic counts for an entry are clipped after reaching

9999.

Note: Horizontal display pages 4 and 5 do not display with an NRD525.

Note: In displaying time plots, if blanks are seen rather than --__-,

the scan start time (set in the Overall screen) is set outside

the allowable sample window.  The amount of time covered in a

time plot is a function of the sample interval.  For example,

with a 5 minute interval, the 128 sample limit will span 5*128

minutes or 10.67 hours.  If the start time was 0 hours, samples

past 10.67 hours will not display.

"ALT-B" for "Bank" - used to switch radios.  Use this command if you specified

that a second radio was to be controlled and you wish to change the

active radio.  This command is for fanatics with clearly too much

invested in this hobby...

"ALT-M" for "Move - used to move the contents of the active radio to the

inactive radio.  (See also the ALT-B command above for switching radio

"banks".  Useful in conjunction with scanning or spectral plotting to

continue listening to a station (say for an ID) while proceeding with

the primary radio.

"ALT-D" for "Decimal" - (NRD535 only) toggles between 2 and 3 decimal

frequency display.  (Default is 2 decimals)

GRAPHICS COMMANDS:

For users with NRD535s and with graphics hardware, there is a G(raphics

option.  Most graphics hardware should work.  Testing included VGA, EGA,

and CGA.  The following adapters should work but were NOT tested:  MCGA,

EGA64, EGAMONO, HERCMONO, and PC3270.  This code requires reading the radio's

S-Meter which is not possible on the NRD525 and hence does not show up on the

menu for the 525.  From the 535 command menu, select:

"G" : for "Graphics".  Selecting the Graphics option produces the following

command menu:

GRAPHICS:  C(lear, T(ime plot, S(pectral plot, Q(uit

These commands are as follow:

"C" for "Clear":

Clears the graphics screen.

"T" for "Time plot":

Produces a time domain plot of the S-Meter output.  This is useful

for watching signals fade in and out over time.  Units are in dB

and calibrated to the NRD535 S-Meter.  Hitting the SPACE BAR will

stop the plotting and return control to the Graphics menu.  The

x-axis is labled in UTC as the plot is created.  The plot will

automatically scroll to the left after the screen fills up.

"S" for "Spectral plot":

Produces a frequency domain plot of the S-Meter output over a user

specified frequency range.  This can be useful for a number of

things like analyzing interference effects and serious DX bottom

fishing.  There are really two modes of spectral plotting

described below:  Automatic Sweep and Manual Sweep.  For both of

these, you enter the range of frequencies that you wish to scan.

In Automatic mode, the internal sweep feature of the radio is

used.  In Manual Sweep, you specify the frequency step for each

sample, the number of samples per data point and an AGC settle

time allowance.  Both modes are useful and both successively

average data across sweeps to make signals stand out above the

noise.  Automatic Sweep mode produces the greatest level of detail

per unit time.  Manual Sweep mode can be much faster if larger

frequency steps are used.  For example, to sweep 100kHz with 5kHz

steps with nominal sample and agc values requires around 45

seconds versus 7.5 minutes for automatic sweeping.  Manual mode

also provides a greater degree of control over sample amounts and

settling time.  The sampling for both modes is done in CW mode

using Narrow bandwidth (1000Hz).

(Author's note:  After much use, I find manual mode to be MUCH

faster and useful and will probably remove automatic mode in the

future unless I receive requests otherwise.)

Automatic Scan Mode:

You will first be prompted to enter the start and stop

frequencies.  The program defaults to -/+ 5khz off the current

frequency if you wish to hit enter without typing a number for the

start and stop frequencies.  You will next be prompted to enter

the frequency step.  Entering 0 results in an automatic sweep.

In this mode, the program will start the radio to sweeping and

will poll the radio S-Meter at regular intervals.  These S-Meter

readings are plotted as points and the process of scanning the

specified range will continue until you enter another command.  A

running average for each frequency is maintained and this can be

displayed (see the Average command below).  You will be prompted

to enter "the # of sweeps in the running average".  This in

effect sets the "frequency response" for the running average.

Selecting "1" will do no averaging (each point will be plotted as

returned from the radio).  Selecting "100" will cause new points

to be effectively averaged with the last 100 samples.  (This is

built up over time.  The second sweep will be an average of 2,

the third an average of 3 and so forth up to 100 in this

example).  Larger values help pull signals out of the noise but

will limit the plot's response time to changing conditions.

Manual Scan Mode:  (Preferred mode)

Entering a non-zero number for the frequency step in the above

process initiates Manual Scan Mode.  After entering a non-zero

step, you will be prompted to enter the "# of sweeps in the

running average" described above.  You will then be prompted to

enter the number of samples per point plotted (a number between 1

and 256).  A value of "1" produces the fastest plot.  (Subsequent

sweeps are always averaged so "1" is usually the best choice).

You will next be prompted to enter the "# of sweeps per history

sample".  If your system has sufficient memory, successive sweeps

are stored in memory in compressed form for later play back.  (See

the History command described later).  The default of "1" will

save every sweep, 2 saves every other sweep, etc.  Generally the

default is best but larger values can be used for extended

sampling periods.  Finally, you will be asked to enter the "AGC

settle allowance".  Simply hit enter to use the default value or

change this value up or down to allow a greater delay after

frequency changes before real sampling begins.  (Valid entries are

any number between 0 and 255).  Use "0" for fast plots if you do

not mind the slow AGC decay.  "20" is a reasonable overall value

and is the default.  (Note:  An AGC mod that consists of adding a

390K resistor from the "+" side of C104 to the "-" side of C105 on

the IF Amp Board greatly reduces the decay time and allows a value

of "0" to work well for this command.  Sweeps on my computer run

about 6 times faster with this change.  Do any receiver

modifications AT YOUR OWN RISK!)  S-Meter readings are displayed

as point during sampling at a given frequency and then the average

for that frequency is plotted as a solid vertical line.

Successive scans are averaged.  If you have selected only one or a

few sample points per frequency, you may not notice the individual

sample point plots.  This is normal.

Note: A special mode exists for entering the frequency range for

Spectral plots.  Entering CTL-S (for select) at the prompt

for the start frequency calls up a template of frequency

ranges.  Moving the cursor to one of these bands and hitting

ENTER will select this band for the plot to be plotted.

These band selections are contained in a file "band.txt"

which can be modified for your preferences.  The format is

start frequency (leading spaces allowed), any number of

spaces and the ending frequency.  Comments after the ending

frequency are ignored but at least one space must separate

the ending freuency from the comment.

The following command prompt is displayed when doing Spectral

plots:

COMMANDS: A(verage, C(lear, F(ind, R(eset average, Q(uit

Hitting "A" for "Average" will plot the average value for each

frequency.  Typically, after sampling a while, this is useful

to see signals that might otherwise be in the mud and to filter

out lighting and modulation effects.  A running average is

maintained that weights each sample equally. This is mainly

useful in Automatic Scan mode since each Manual Scan sample is

already displayed in average form.

"C" for "Clear"  clears the screen but does not stop the sampling

or reset the statistics used in averaging.

"R" for "Reset average" resets statistics used in averaging.  This

is useful if conditions are changing rapidly.

"Q" for "Quit" returns to the Graphics menu.

The "F" for "Find" command is a special mode that stops scanning

and allows you to tune the radio either manually or with the

computer.  This mode allows switching back and forth between the

main database screen and Find mode.  After selecting "Find", the

radio will cease scanning,  the radio state will be returned to

its state prior to entering scan mode, the average Spectral

response previously computed will be plotted, the radio's current

status will be displayed at the top of the screen, a green cursor

will mark the receiver's frequency on the Average plot, and you

will see the following prompt: (whew!!!)

FIND: C(ontinue, L(og, H(istory, N(ext hit, P(rev hit, Q(uit

"C" for "Continue" returns to the Spectral scan mode and resumes

sweeping the receiver.

"L" for "Log" returns to the main log screen.  Hitting "G" for

"Graphics" from the log screen returns to "Find" mode.  This is a

very powerful command.  You can tune the radio in "Find" mode to

some interesting peak, toggle back to the "Log", hit the "*"

command to find the selected log entry (if this is sorted in

frequency order).  Further, you can tune around in "Log" mode,

toggle back to Find mode and the cursor will now be placed on

the new frequency (if it was in the sweep range).

Try this command and use the radio controls to change the

frequency.  You will see the cursor move across the screen as you

tune and will see a frequency display change at the top of the

screen.  The radio can also be controlled from the program in this

mode.  The left and right arrow keys move the frequency up and

down one sample point at a time.  The TAB keys move this by 25

sample points.  The + and - keys move the frequency by +/- 5 khz.

The usual mode and bandwidth control keys function normally in

this mode as well.  The ALT-A, ALT-F, ALT-M, CTL-J and CTL-N keys

from the main prompt are supported in "Find mode" as well.

Hint: the +/- commands always tune to an exact multiple of 5 khz.

These can make tuning in an exact frequency much easier than the

TAB or cursor keys.

"*" does the equivalent of executing the "L" command followed by

the "*" command.  One keystroke takes you from the graphics find

mode to the database area and locates the cursor on the entry

most closely associated with the radio's frequency.

"H" for "History" displays the Spectral Plot History buffer of

successive scans in movie fashion.  This history data is built up

automatically during Spectral Plots.  The sample rate is

determined by the # of sweeps per history sample value specified

when the Spectral Plot was run.  If a running average interval of

10 was specified, every 10 sweeps, a snap shot of the data will be

taken.  The total number of sweeps that can be stored is a

function of your available memory.  My 386 system with DOS 6.0 and

Windows 3.1 has slightly over 610K for applications and can store

slightly more than 140 frames.  Once all the available frame

memory is used, the program will continue to sample by replacing

the oldest data with the new scan.  This mode is very useful for

observing propagation effects and observing trends that would not

otherwise be obvious.  The prompt for this mode looks like:

History: S(top, G(rid off, Q(uit

Hitting "S" for "Stop" will freeze the action.  Hitting the "+"

and "-" keys will cause the history frame to advance or retreat

one frame at a time.  Hitting "S" again resumes the automatic

advance.  "G" for "Grid off" tells the program to not spend time

redrawing the 5khz and dB markers.  This option is useful to speed

up the draw rate at the expense of "holes" in the grid.  Typing

"G" again restores the normal mode.  "Q" for "Quit" returns to

"Find mode".  If selecting History doesn't do anything, either a

scan has not yet been saved or your system has insufficient memory

to save scans "Q" terminates Find mode and the Spectral plot and

returns to the main Graphics menu.

"N" for "Next hit":  Finds the next peak in the graph and tunes the

radio to this frequency.  This is useful for rapidly skipping

through a scan.  The program automatically converts the frequency

found to the nearest integer equivalent (e.g. 15070.4 to 15070)

and to the nearest 5 khz if the range scanned is 100 khz or

greater.  This conversion compensates for the granularity of

sampling and agc effects which become more pronounced with greater

scan ranges. The utility of this command depends on signal

conditions and the selected sample rate.  For example, using a

sample interval of 5 khz will not produce local maxima on

broadcast bands that correlates with the presence of a signal.

"P" for "Previous hit":  The reverse of the "N" command.

"Q" for "Quit": Returns to the main menu and exits Graphics mode.

DATABASE COMMANDS:

This program allows for a set of up to 84 separate databases.  These are

selected with the Journal command (described later).  One database is displayed

at a time.  The status line at the bottom of the display will show the "active"

log.  When you switch logs, the program remembers the previous log which is

shown as the "inactive" log.  Commands described below allow hot keying between

these logs and copying data from the inactive log.  A highlighted line shows

the position in the database.  Cursor control keys move about in the database

as do "PgUp" and "PgDn" keys.  "Ctrl-PgUp" and "Ctrl-PgDn" keys page by 10

pages at a time.  The "HOME" key takes you to the top of the database.

Likewise the "END" takes you to the end.  The following commands affect the

"active" log.

"L" for "Log":

"Logs" a new entry in the "active" database.  Time stamp and radio contents

are provided automatically.  User is placed in "Edit mode" at the call

sign location.  Enter the call sign and hit the "tab" key to enter the

location.  Hit the "tab" key again to enter a comment.

Note: This comment field is contained on a different "page".  (See the

"Page" command).

A special "CTL-S" (for Select) command (after hitting "L" for "Log"

enters a mode where the station ID and location can be selected from an

existing list of stations.  After hitting CTL-S, Just move the cursor to

the appropriate entry and hit the "Enter" key to accept this entry (or

the "ESC" key to return to the manual approach.  Typing letters in

"select mode" automatically finds the first entry matching the pattern

entered.  For example, Typing "r deu" would skip to R. Deutche Welle.

The file "TEMPLATE.TXT" may be edited to add or change entries in this

list.  Be sure to use an editor that does not insert special characters

in the file).

Note: The template file may be edited to add other fields to override

program defaults.  These must be entered in the specific column

numbers indicated below:

001..019  Station ID

040..066  Location

070..139  Comment    [default is blank]

140..143  Start time [default is current time]

145..148  End time   [default is current time]

150..155  Date       [default is day's date]

"C" for "Confirm":

"Confirms" and existing log entry.  Date is updated to current date.  If

time is outside "Start" and "End" time fields, these are changed.  The

contents of the radio are polled and this data is updated.  (Particularly

valuable for drifting stations).

Note: The time algorithms are rather simple minded.  Mostly they do the

right thing.

"T" for "Tune":

"Tunes" the radio from the highlighted database entry. "5" is an alias for

this command to cluster this on the keypad with "+", "-", "*", etc.

"ALT-L" for "Last":

Restores radio setting prior to the last "Tune" or "set Frequency"

command.  Also useful for toggling back and forth between 2 frequencies.

"*"  This command finds the first database entry with the same (or higher)

frequency that the radio is set to.  Sort of the opposite of the "Tune"

command.  "8" is an alias for this command for keyboards without an

unshifted "*".

"E" for "Edit":

Because the program uses commands, you cannot just type over database

entries without using this mode.  The edit field will be highlighted and

the entry can be changed.  "Home" takes you to the beginning of the field;

"End" takes you to the end.  The "insert" and "delete" keys work.  When

finished, hit "enter" or use a "tab" key to bounce to the next field.  The

ESC key cancels the edit and restores the field contents prior to the

edit.

"P" for "Page":

Each database entry is displayed horizontally over three "pages".  Hitting

the "P" key toggles between these three.  Cursor and Tab keys can be used

as well.   Alt-P acts as a back page command.

"D" for "Delete":

This removes an entry from the database.

"N" for "uNdelete":

Reverses accidental deletions.  Since subsequent "loggings" use up deleted

entries, this is best used immediately after an accidental delete.

"S" for "Sort":

The database can be sorted in ascending order with any two fields.  For

example, by frequency and by time.  In this example, the database will

be sorted by frequency with all stations with a common frequency listed

in time order.

Note:  For automatic tracking of the receiver frequency (see "*", "+",

and "-") to work properly, the database should be in frequency

order.  Automatic tracking can be disabled with the Overall command

if desired.

Note:  This only sorts the active log.  Different logs can be sorted in

different ways.

"M" for "Mark range":

Marks the current highlighted line.  Moving the cursor to another line and

pressing "M" again marks a range of lines.  Multiple ranges may be marked.

Marked lines are displayed in yellow.  See the Journal, "+", "-" and "X"

commands for usage.

"CTL-M" for "Mark line":

Marks the current hightlighted line.  Similar function to the "Mark range"

command but used to mark individual lines as opposed to ranges of lines.

This function is accessed by depressing the CTRL key then pressing "M".

"U" for "Unmark":

Unmarks all marked lines.

"CTL-L" for "unmark Line":

Unmarks the current line only.  This is useful for excluding individual

entries from a range of marked lines.

"CTL-R" for "unmark Range":

This command is the logical opposite of the Mark Range command.  Unmarks

the current highlighted line.  Moving the cursor to another line and

pressing "CTL-R" again unmarks a range of lines.

"CTL-D" for "Delete marked entries":

This command deletes all marked entries.

"CTL-S" for "Search":

Search is used to search the database entries for a pattern.  When this

command is selected, you will be presented with the following prompt:

Search: select class:  N(ext, M(ark all, Q(uit

"Q" for "Quit" cancels the command.  "N" for "Next" will find the next

occurence of a pattern starting with the current highlighted line.  "M"

for "Mark all" will find all occurences of a pattern and mark them for you

(see the Mark command).  You will next be prompted to enter the field to

search and then to enter the pattern you wish to search for.  For example,

if wanted to mark all entries for the Voice of America, you would first

unmark any already marked lines with the "U" command.  (Unnecessary unless

you've marked some some lines already).  Next select the Search command by

pressing CTRL and "S" at the same time.  Hit "M" for "Mark all".  Hit "I"

to select the station "ID" field for the search field.  Next enter "voa"

for the search pattern.  (Note:  the case of a search pattern is ignored.)

The program will acknowledge your selection by placing the following

prompt and message:

Hit <SPACE BAR> to start search: A(nd conditional, O(r conditional, Q(uit

"VOA" in <ID>

Hitting the "SPACE BAR" initiates the search.  If you desire to add

additional search qualifiers, use "A" for "And" or "O" for "Or".  Using

"And" qualifiers means that the first pattern AND the second pattern must

be found for a match to occur.  "Or" means that the first pattern (or

series of patterns separated with "And") OR a second pattern must be found

for a match to occur.  For example, suppose we wish to mark only those

entries for the VOA in Russian and originating in Greece or the UK.  To do

this, hit "A" for "AND" and repeat the previous steps to select "Location"

and "Greece".  The program will then prompt with:

Hit <SPACE BAR> to start search: A(nd conditional, O(r conditional, Q(uit

"VOA" in <ID> & "GREECE" in <LOCATION>

You then repeat the process by selecting "A", the "Comment" field and

typing "Russian".  The program will then prompt with:

Hit <SPACE BAR> to start search: A(nd conditional, O(r conditional, Q(uit

"VOA" in <ID> & "GREECE" in <LOCATION> & "RUSSIAN" in <COMMENT>

You will next select "O" for "OR" and repeat the same process of

entering the ID, Location, and language.  The program will now prompt:

Hit <SPACE BAR> to start search: A(nd conditional, O(r conditional, Q(uit

"VOA" in <ID> & "GREECE" in <LOCATION> & "RUSSIAN" in <COMMENT>

OR "VOA" in <ID> & "UK" in <LOCATION> & "RUSSIAN" in <COMMENT>

At this time, you hit the "SPACE BAR" and the program will then

find all occurences of "VOA" in the station ID field meeting these

conditions.  For an entry to qualify, all "AND" conditions must be met

per OR grouping. See also the "CTL-Again" command.

Note: Using combinations of the Search & Mark command and other display

options can be quite powerful.  For example, use the "X" command to

display only the stations found.  Use the "Display only active" mode

(set in Overall command or toggled with the "Z" command) to see targeted

stations that are currently active.

Note:  Frequency, Time and Date search values can be preceded with ">",

"<", ">=" or "<=" to create a range of acceptable values.  For example,

to mark all frequencies less than 15khz, enter: "<15000".

"CTL-A" for "Again":

This command re-executes the last "Search" command.  This is used to

rapidly find the next occurence of a pattern without having to re-enter

the the search qualifiers again.

"V" for "moVe":

This command moves data from marked lines to sequential memory channels.

After marking the desired lines and selecting this command, you will be

prompted to enter the starting channel number to write to.  The program

will then search for marked entries and write the contents to the radio's

memories.  Radio channels are filled starting with the selected channel

and moving up.  The process will continue until all marked lines have

been written or channel 199 is reached.  Hitting the ESC key when prompted

for the channel number cancels this command.  Hitting ENTER without

entering a channel number defaults to the next higher channel based on the

last moVe command.  (The program is initialized with this being outside

the channel range to prevent accidental channel writes.)

Note:  For the NRD535, the program uses channel 199 as a scratch pad and

data stored in this channel will be overwritten.

Note:  For the NRD535, entering 999 as the channel number will clear all

channels.

Note:  Successive marked entries with redundant radio settings (freq,

mode, bandwidth and agc) will not be sent to the radio.

"ALT-I" for "Id":

This command tags an entry with an "*" after the time field.  Hitting

ALT-I again for an entry removes the tag.  This command was requested by

users of existing databases wishing to mark a station as having been

heard.

"ALT-K" for "Klear Id"

This command clears all Id tags set with the ALT-I command.

"CTL-C" for "Copy Id"

Marks all entries with Id tags set with the ALT-I command.

"H" for "Help":  Type for help on using this program.  This yields the

following prompt:

Help: A(ll cmds, R(adio cmds, D(atabase cmds, M(ark cmds, P(rint, Q(uit

Hit the "A" key to display a summary of all commands.  The PgUp and PgDn

keys work to scroll through this list.  The "R" key displays commands

specific to controlling the radio.  Likewise the "D" key displays commands

associated with the database portion of the program.  The "M" key shows

the database commands associated with the various Mark features for quick

reference.  "P" will send the screens associated with the "A" key to the

printer.  "Q" returns to the main program.

"/":  Selects alternate command prompt.  All commands are always active, the

prompt only serves as a reminder.

"A" for "Alternate":

This command allows hot keying between the active and inactive database.

This will have no effect if there is no inactive database.  (The active

and inactive databases are displayed in the Program Status line at the

bottom of the screen).  This is particularly useful for switching between

a station log and a target log.  The "A" key was selected for its

convenient keyboard position.

"W" for "Write":

This command will "write" the log data from the highlighted line for the

"inactive" (not seen) log to the highlighted area for the active log.

To illustrate how the "A" and "W" commands might be used, consider this

example:  It is dawn and you are using your "INDO TARGET" log to search

for new stations.  You find one!  You use the "Confirm" command to update

the time and date in the target log and use the Journal command to bring

up the log you use to keep confirmed catches.  To save time, you don't

bother to find the "proper" place in this log for this station (you can

sort it later).  You hit the "W" for "Write" key and the entry appears.

Now hit the "A" for "Alternate" key and you are back in "INDO TARGET".

Find another one?  Confirm it and just hit "A" for "Alternate" again to

toggle back to the main log and use "W" again.

Note: This creates a new entry and does not overwrite data.

Note: The Journal "Read", "Write" and "Move" commands may also be used.

"ALT-U" for "Upload radio channels to database":

This command will automatically take the contents of a range of channels

from the radio (535 only) and create log entries for them.  You will be

prompted to enter the start and ending channels and the program will do

the rest.

Note: this command requires a version F or later PROM.  If the Overall

Screen (see the Overall command) says: "values cannot be read" and no

PROM version is displayed, you should contact JRC to upgrade the PROMs.

"J" for "J(ournal"

This command will display the various log choices and its own command

menu:

Journal: S(elect, C(reate, D(el, R(ename, E(xport, I(mport, P(rint, Q(uit [/]

These commands will now be described:

"S" for "Select":

Select a new active database.  You will be prompted to enter the

name OR the number of the database you which to view.  (Note: This is

a change.  Earlier versions required entering the name).   This new

database becomes the active database and the last one becomes the

inactive one.

"C" for "Create":

Creates a new, empty database.

"D" for "Delete":

Deletes a database.

"R" for "Rename"

Renames a database

"E" for "Export":

Exports the selected database.  This command writes the currently

selected database to a file in ASCII format.  This format was chosen

over other data exchange formats since text editors are more common

than databases and spreadsheets and since these allow provisions for

importing text formats.  Since the "Import" command can also read this

format, these commands should be useful for exchanging target files

and logs among users.  Files can be any suffix but the default is

".LOG".  (I also use this command to create backup copies of my logs

from time to time).

Note: Export copies all records currently displayable to the file.  The

"X" and "Z" commands can be used to limit what information gets

displayed and hence exported.  Care should be taken when using

"Export" to do backups to insure some records are not hidden.

"I" for "Import":

Imports data from another database.  Currently, two formats are

supported.  After selecting the "Import" command, the following prompt

will appear:

Import: A(scii, N(ew TRS format, O(ld TRS format, Q(uit

ASCII File Import:

Selecting the ASCII format allows importing files of the same format

as created by the "Export" command.  This format is column sensitive

and has the following fields (separated by a space):

Column   Field        Comments

1     date         Use YYMMDD format

8     start time   Use HHMM format

13     stop time    Use HHMM format [optional]

18     frequency    Use xxxx.yy

27     station id

47     location

75     comment      [optional]

145     mode         [optional] use single character

R = RTTY

C = CW

U = USB

L = LSB

A = AM

F = FM

X = FAX

u = ECSS-USB (NRD535)

l = ECSS-LSB (NRD535)

blank is assumed to be USB

147     bandwidth    [optional] use single character

N = NARROW

I = INTERMEDIATE

W = WIDE

A = AUX

blank is assumed to be INTERMEDIATE

149     agc          [optional] use single character

F = Fast

S = Slow

O = Off

blank is assumed to be Slow

151     attenuator   [optional] use single character

Y = Yes

N = No

blank is assumed to be No

Columns past 75 are optional and can be truncated with less than full

line width (assuming defaults are acceptable).  One simple way to keep

the columns straight is to create a file with the "Export" command and

use this file as a template.  Files can use any suffix but the default

is ".LOG".  Use this command to share databases or to map data over

from other databases, spreadsheets, or text editors.

Note:  Make sure you save the file without TABS or other control

characters.

ASCII files available:

Other database files are available as FREEWARE (on PicsOnline/GEnie)

under NRDDTxxx.ZIP where xxx is the version number.  These database

files go beyond the SAMPLE included with the program.  I include in

this package some nifty files that Thomas Mann sent me (Tom

transcribes net logs for ANARC), my personal log files and will

include other goodies as these become available.  A caveat about my

personal databases:  These are meant as examples and not definitive

Short Wave databases by any means!  See also the SWSKED section below

to import data from Tom Sundstrom's English language database for

subscribers and his IS a definitive database.

SWSKED Format File Import:

This option allows importing Tom Sundstrom's English language

database.  To import his database to NRD, copy the "SWSKED.DBF" file

into the NRD subdirectory and select and select "J" for "Journal"

then "I" for "Import" then "S".  This file is NOT included and must

be purchased from TRS Consultants.  To receive more information on

Tom's program, send a #10 SASE to TRS Consultants PO Box 2275,

Vincentown, NJ 08088-2275, USA or FAX (609) 859-3226 or call the Pics

OnLine!  BBS on (609) 753-2540 (HST) or (609) 753-1549 (V.32) .  Tom

offers an update service.  THIS PROGRAM ASSUMES THAT YOU HAVE

COMPLETED THE UPDATE PROCEDURE HIS DOCUMENTATION CALLS FOR PRIOR TO

USE.

"P" for "Print":

Prints either all lines or the marked lines of the current database in

compressed print to LPT1:  (See Mark, Unmark, CTL-M and CTL-L commands

for marking lines.)  Location and Comment fields are truncated to fit

all information on one line.  The last two characters represent the

Mode and Bandwidth.  (See the ASCII inport format for the

abreviations used.)

"W" for "Write": (See menu 2 with the "/" command)

Writes marked lines to database you select.

"M" for "Move":  (See menu 2 with the "/" command)

Like "Write" but deletes the marked lines from the source log after

copying them to the new log.

"Q" for "Quit":

Leave Journal and return to database.  Currently same as Select

command.

USING DATABASES:

Starting with version 4.0, database samples are now stored in a separate file

(NRDDTxxx.ZIP where xxx is the version number) to save download time.  To load

a sample database: start NRD; select "J" for "Journal"; "I" for "Import"; "A"

for "ASCII" and enter the name of one of the .LOG files (like LOG.LOG).

I include my personal log (called, somewhat unimaginatively "LOG") as a sample.

In practice, I use this to record stations as I hear (or re-hear) them and use

other logs for specific target areas.  You can very quickly tune through a list

of INDOs at dawn by creating a special target file, for example, and it's easy

to copy entries from one database to another.  You will probably want to start

over with your own log.  If you do some programming and have a computer log in

some other format, it's not hard to map over using the import ASCII function.

IN CASE OF DIFFICULTY:

Problems can be caused by using the wrong receiver baud rate.  The program uses

1200 baud for the 525 and 4800 baud for the 535 to communicate with the radio.

(NRD535 users need not worry about this - the 535 uses only 4800 baud).  (To

determine the baud setting on a 525, hold down the "MEMO" key and hit the "8"

key on the Radio keypad.  The radio should display "1200" on the frequency

readout.  If it shows "300", press "MEMO" "8" again.  The most likely source of

difficulty I've seen to date is with the cable setup.  To isolate problems, it

is sometimes useful to use a communications package, such as PROCOMM, and use

it to send "I1" to the radio.  You should see the radio REMOTE light come on

and see a string of data come back from the radio.  If the REMOTE light comes

on but no string returns, the cable should be examined.  If the REMOTE light

does not come on, it could be you are selecting the wrong COM port or have some

other problem (including the cable).

MICROSOFT WINDOWS / IBM OS/2

NRD can be used with Windows and OS/2.  I use NRD in one window and the

wonderful GEOCLOCK program in another.  (See NASWA Listener's Library May 1990

for John Bryant's review on GEOCLOCK -- get this program and register it in

order to get all the extra maps!!!).  The file NRD.ICO and GEOCLOCK.ICO are

icons for use with windows.

PERFORMANCE HINTS:

Using Disk Cache programs really help.  As you delete and add entries over

time, the database will become fragmented and you may notice some degradation.

To remedy, first back up the data files (using DOS), then mark all entries (put

the cursor on the top line and hit "M" for "Mark", go to the last entry and hit

"M again -- all lines should be Yellow now if you have a color monitor).  Next

hit "J" for "Journal", Create a new database ("C" for "Create), and select "W"

for "Write".  When the program asks for "Log to write to:", enter the name of

the database you just created.  Next delete the old database.  This process

copies the old log to the new one and eliminates any fragmentation.  In

practice, this process is rarely needed.  (The ASCII file Export and Import

features can also be used to defragment files.)

Note:  The defragmentation hint above is different than disk fragmentation

which can also affect performance.  Using commercial disk defragmentation tools

such as those included in the Norton Utilities is helpful when the disk becomes

fragmented.

UPDATE SERVICE:

The PicsOnline!  BBS ((609) 753-2540 (HST) or (609) 753-1549 (V.32)) and GEnie

are probably the best places to get new versions.  Other BBS services and

Compuserve have NRD but only as transported by others so often do not have the

latest version.  For folks who prefer the mail, I am glad to make and mail

copies provided that you send a diskette and a mailer stamped and addressed

with your address.  (Including this in a business envelope works nicely.) Users

wishing notification of future releases can send me a SASE.  After receiving a

notice, send another SASE to continue to receive notification.  Please do NOT

send money!

FEEDBACK PLEASE:

I'm interested in feedback on the program and ideas for how to make it better.

Many of the features in NRD are a result of suggestions from others.  I can be

contacted at:

Tom Whiteside (512) 258-5924

11505 Oak View

Austin, TX 78759-4638

I can also be reached on GEnie (T.WHITESIDE).  Hope you enjoy NRD!