SETTING UP YOU MT224EH FOR HIGH SPEED OPERATION

July 16, 1988

Version 10

A. PURPOSE

=======

These  instructions will help you  get the most out of your  new

MultiTech  MT224EH modem by  enabling RTS/CTS flow control.  You

will be able to do the following:

use MNP level 5 protocol for error free communication

use  data compression to communicate at data rates from 5%

to 100% faster

use the new protocols I-modem and Y-modem-G that depend on

the modem doing error correction

configure QMODEM 3.1A, PROCOMM Plus 1.0 or TELIX 3.00

In  order to use data compression, you must transmit data to the

modem faster than it transmits data on the telephone line. These

instructions  set your serial port  for 9600 baud and the  modem

for 2400 baud.

In order to for the modem to re-transmit garbled blocks of data

or  to  send out compressed  data at 2400 baud while  receiving

data  from  your computer at 9600  baud, it must have a way  of

telling  your  communication  program to stop transmitting  new

data. For this the modem uses RTS/CTS flow control.

"The MultiTech 224E is one of the most popular 2400 baud modems

that  incorporates  MNP  error correction into  its  circuitry.

Setting  the various hardware and software parameters to enable

MNP  and  CTS/RTS  flow  control is somewhat tricky,  and  this

document is intended to simplify that task."

MultiTech publishes a fine reference manual that describes all

the switches and options available. You should read it for all

the details.

Unfortunately,  the manual does not describe the COMBINATION of

all  switches which must be set for effective MNP level 5  use,

nor  does it say how to install popular communications software

to  take advantage of MNP  level 5. Therefore, I have  provided

these instructions for a first time user who may be overwhelmed

with the choices in the manual and need a cookbook like this.

"The  instructions are for the MT224EH models that have Class  5

MNP.  For those older models that incorporate Class 3 MNP  only,

some of the software switch settings will generate errors. These

will  not affect modem operation. (MultiTech does have a program

to  upgrade  Class  3  MNP  to  Class 5 MNP;  contact  them  for

information.)"

B. CAUTION

=======

In order to set up your system to take advantage of MNP level 5

you  must do ALL, repeat  ALL of the following as described  in

this document:

1. have a correct modem cable

2. set the 8 hardware DIP switches under the MT224EH

3. set the 4 hardware DIP switches inside the MT224EH

4. set the 3 hardware jumpers inside the MT224EH

5. store MT224EH software configuration parameters

6. configure your communications program

You  cannot  do only some  of these steps. All  parameters  and

settings  must be MUTUALLY CONSISTENT for communication to take

place.  If  after following  the directions below, your  system

doesn't work, please double-check everything.

C. CORRECTIONS AND FEEDBACK

========================

I appreciate your feedback on how to improve this document.

Thanks.

Paul Thompson

612-853-6062 (computer answering device)

leave a message on John Friel's QMODEM board 319-233-6157

or on the MultiTech board 612-631-0922

D. ACKNOWLEDGEMENT

===============

These instructions are an extension of a document originally put

together by:

S. David Klein

who  writes, "These settings have been tested using QMODEM  SST

3.1a  with  both  the  I-Modem  and  Y-Modem-G  protocols  with

artificially injected noise on the phone line, and we have been

unable  to  produce  a  bad  file  transfer.  With  a  Class  5

connection  (CONNECT RELIABLE COMPRESSED), using the  alternate

settings given [below], we have achieved throughput of over 650

cps  on an un-compressed document file, and over 290 cps on  an

ARC  file, a better than 20% improvement over normal 2400  baud

Class 3 connections."

(Quoted text is from David's document.)

E. PRE-CHECK

=========

Before using the MT224EH's advanced features, first see if the

normal set-up works.

Attach  the MT224EH to your computer. Leave all settings as they

come  from  the  factory.  This includes hardware  switches  and

jumpers  as well as software  switches stored in RAM inside  the

unit.  (You can reset  the software switches to factory  default

with  the command AT&F&W0.  If the hardware switches or  jumpers

have been messed with, check them carefully.)

Follow  the  directions  in  QMODEM  SST 3.1A or  PROCOMM  Plus

documentation for installing a Hayes modem. Test communications.

Hopefully,  everything is fine and you are now ready to move  on

to setting flow control.

(There  are many things  that can go wrong with  communications:

bad  cable, using wrong port,  two serial cards set to the  same

port  address, wrong DIP switch settings, bad set-up strings  in

your  communications program, etc. It just makes sense to  start

from  a known situation; and there is more help for you if  your

starting point is 100% Hayes emulation.)

F. CHECK YOUR MODEM CABLE

======================

Even  though your cable may  have worked for standard Hayes,  it

may  not  be correct for  flow control. Some cables  have  fewer

pins,  or  strap  certain  pins  together.  Your  communications

program  must have the ability to read the true state of CD  and

CTS  and  to control RTS  and DTR. All lines should be  straight

through. See page 3-7 of your MultiTech user manual.

This set-up requires:

pin 2 SD Transmitted Data

pin 3 RD Received Data

pin 4 RTS Request to Send (used for flow control)

pin 5 CTS Clear to Send (used for flow control)

pin 6 DSR Data Set Ready

pin 7 SG Signal Ground

pin 8 CD Carrier Detect (used by communications program)

pin 20 DTR Data Terminal Ready (used by your program to hang-up)

pin 22 RI Ring Indicator

G. SETTING THE MULTI-TECH 224EH HARDWARE SWITCHES

==============================================

"The MT224E has two banks of DIP switches and three jumpers. One

bank of eight switches is accessible underneath the modem. These

8  option switches duplicate the Hayes 1200 switches. The  other

bank  of  four  switches  and  jumpers are  accessible  only  by

removing  the cover of the modem. (Refer to your owner's  manual

pages  9-1  to 9-13 for  instructions on opening the  modem  and

locating the switches and jumpers).

"The  following  settings  will  enable  MNP  error   correction

(providing  the modem that is called is so equipped) and CTS/RTS

flow  control. They will  also enable the full verbose  response

set  of the MT224 which will keep you apprised of what the modem

is  doing,  and  will  wait  for dial tone before  dialing.  The

setting  of the bank of eight switches should be as follows  (up

refers to the OPEN position):

1-UP (DTR normal),

2-UP (verbose responses),

3-DOWN (enable command response),

4-UP (enable command echo),

5-UP (enable auto answer),

6-UP (CD & DSR normal),

7-UP (single line RJ11), and

8-DOWN (enable command mode)

"All  of  these  are  the  factory  default  settings  with  the

exception of switch 1, which is reversed.

"The  setting of the bank  of four switches [located inside  the

unit  behind the front panel] should be as follows (up refers to

the OPEN position):

1-DOWN (CTS normal),

2-UP (Dial-up operation),

3-UP (wait for dial tone),

4-DOWN (asynchronous operation).

"Switches  2  and 4 are  factory default, switches 1 and  3  are

reversed.

"The jumpers should be set as follows:

RTS normal (not forced),

CTS to follow RTS,

DSR to follow CD.

The  CTS  and  DSR  jumpers  are  factory default;  the  RTS  is

reversed."

[Viewing  the inside of the  MT224EH from above with the  front

panel on the right, the jumpers should be set as follows:

---------------------------------------------------------------

|                                                             |

|                                 CTS/RTS                     |

|                                    ===                      |

|                                  * * *                      |

|                                                             |

|                                                        DUUD |

|                                                        4321 |

|                                                             |

|                                                             |

|                                                             |

|                                                             |

|  ===                                                        |

|* * *                                                        |

|D                                                            |

|S                                                            |

|R                                                            |

|                                                             |

|                                                             |

|                                                             |

|                     R                                       |

|                     T                                       |

|                     S                                       |

|                 ===                                         |

|                 * * *                                       |

|                                                             |

|-------------------------------------------------------------|

H. SETTING THE MULTI-TECH 224EH SOFTWARE SWITCHES

==============================================

The  following software switches should be set. (Chapter 5 and 7

in the manual)

B1 - Wait for dial tone

E1 - Echo on

M1 - Speaker on until connect (can also be M0)

Q0 - Result codes on

V1 - Result codes sent as words

X1 - Extended result codes

&E1 - Auto reliable mode

&E4 - CTS flow control

&E7 - XON/XOFF pass through on

&E8 - ENQ/ACK off

&E10 - Normal mode Modem flow control off

&E13 - Pacing on

&E15 - Compression on

&Q0 - Standard result codes

$MB2400 - Set modem baud rate 2400

$SB9600 - Set serial port baud rate 9600

$BA0 - Baud adjust off

&W0 - Store parameters

&A0 - Answer back off

$A1 - Buffer data during link

&B0 - Normal transmit buffer

&BS1 - Max block of 256 bytes

&C1 - Carrier detect normal

$D0 - DTR dialing off

$F1 - Fall back on CR during link

&G0 - Guard tone off

&I1 - 10 min inactivity timer (can also disable or set longer)

$MI0 - MI/MIC option off

&R1 - CTS forced on

$R1 - do not disconnect after 12 tries (can also enable)

&S1 - DSR normal

&T5 - Remote loop off

&V1 - CCITT V.22 answer tone

&W0 - Store parameters

To set the modem, enter your communications program and without

dialing  (ie  modem  is  in  command  mode/off-line)  type  the

following  lines. The modem will reply OK after each line.  The

&W0  ensures  that your  selections are written to the  modem's

memory  where they are maintained by a 7 year battery. Any time

you  turn  power  on  or  issue  ATZ, these  settings  will  be

activated.

AT B1 E1 M1 Q0 V1 X1 &E1 &E4 &E7

AT &E8 &E10 &E13 &E15 &Q0

AT $MB2400 $SB9600 $BA0 &W0

AT &A0 $A1 &B0 &BS1 &C1 $D0

AT $F1 &G0 &I1 $MI0 &R1 $R1

AT &S1 &T5 &V1 &W0

Use  ATL5L7  to have  the modem display its  current  settings.

Refer to the manual for more detailed explanations.

"A  few  words of  explanation  on some of the  software  switch

settings:

&E1 enables automatic MNP error correction if the modem you

are calling also has MNP enabled.

&E4  enables  the  CTS  flow  control that  the  new  file

transfer  protocols use to control data flow between  your

computer and the modem.

&E7  passes  the X-on  X-off codes through to  the  remote

computer.  A  careful re-reading  of the manual  and  some

limited  experimentation seem to confirm that this setting

has  no effect on transmission when &E4 is set, but I keep

it  on anyway since I've had months of reliable usage with

it set.

&R1  forces CTS initially high so that your communications

program  will  know  CTS  is enabled. The setting  of  the

jumpers  and hardware switches ensure that even with  &R1,

flow control will work."

$R1 prevents a hang-up if someone picks up the telephone

from another extension.

&E13  allows the communications program to tell the  modem

to  stop sending data by dropping RTS. It's not clear that

the communications programs actually use this feature, but

I've allowed for it.

&E15 enables Class 5.

$MB2400  sets the modem to respond initially at 2400 baud.

If  the  remote  modem  is  slower, the local  modem  will

connect at a lower baud rate.

$SB9600  sets the serial port at 9600 baud so you gain the

advantages of data compression (more throughput).

$BA0 turns speed conversion (turn off baud adjust). Set

your communications program for 9600 baud. Do not allow it

to reset the serial port speed.

A  few old mainframes (1200-7-E-1) like Genie get confused

when  my  modem  sends  a  link message to them  and  they

interpret  the  link  message  as  the  user  name  and/or

password.  Before dialing them I turn MNP off (AT&E0)  but

leave  all  other switches  unchanged. After hanging up  I

turn  MNP back on (AT&E1). To make it easy, I assign these

strings to function keys.

A  somewhat "jerky" screen writing is normal when connected

in reliable mode. This is because the modem holds an entire

block  of  characters until  they are received  error  free

before  sending them to the screen in a burst. If you  have

noisy telephone lines this effect will be more obvious. You

can  get  a somewhat  smoother flow by using smaller  block

sizes &BS0.

I. SETTING THE MODEM'S "S" REGISTERS

=================================

ATS0=0    Sets the number of rings before modem answers

(leave default 0, do not answer; your communication

program host/answer mode  command  resets this

to S0=1)

ATS7=60   Sets number of seconds to wait for carrier before

hanging up (Default 30 not enough for long distance

dialing)

AT&W0     Writes these values into modem's memory

(See chapter 6 in the manual.)

J. QMODEM 3.1A INSTALLATION INSTRUCTIONS

=====================================

You  can call the QINSTAL program from the DOS command prompt or

from  within  QMODEM  with  Alt-N. I've taken snapshots  of  the

relevant  installation screens. Rather than tell you what to do,

I  thought I'd show you.  These settings are known to work.  Get

your  program  working with  these settings first;  then  change

later if you want.

1. Qinstall - Modem Communication Parameters

Set  your initial baud rate to 9600 and keep all  communication

to the modem at that speed.

Baud       Data   Parity   Stop

Enter initial modem settings  [9600 ]    [8]     [N]     [1]

-----^

String to send BEFORE changing Baud rate ....... [            ]

String to send AFTER changing Baud rate ........ [            ]

Number of seconds between redial attempts ...... [1    ]

Number of seconds before cancelling a call ..... [45  ]

Number of milliseconds for DIAL pacing ......... [1    ]

Initial COM port your modem is attached to [1-8] [1]

2. Qinstall - Modem Dial and Redial Parameters

Use  the  DTR method to hang  up the modem. Use ATZ to set  the

modem to the parameters you stored in the modem's memory.

Message your modem returns when connected ..........[CONNECT  ]

Four messages your modem returns when Dialing was unsuccessful

[NO CARRIER   ]  [BUSY      ]   [NO DIALTONE   ]   [NO CONNECT]

Text sent to cancel a Redial attempt .................. [DTR  ]

^----

Text received that cancels a Redial attempt ........... [RING ]

Modem Initialization text ... [ATZ{                           ]

^-----

Modem Dial Prefix [ATDT          ]    Modem Dial Suffix [{    ]

Text sent to Hang up the Modem ........................ [DTR  ]

^----

Character (if any) to be sent twice in BINARY transfers [  ]

3. Qinstall - Misc. Parameters   Part 1

You  must turn CTS signal checking on, and set baud rate adjust

off.

Do you want to use XON / XOFF ..... [Y/N]  [Y]   <------

Do you want QMODEM to make noise .. [Y/N]  [Y]

Do you want to add LINEFEEDS to CRs [Y/N]  [N]

Do you want MUSIC played when found [Y/N]  [Y]

Do you want CTS signal checking ... [Y/N]  [Y]   <------

Do you want Redial to set Baud rate [Y/N]  [N]   <------

Swap the BS to DEL on Startup ..... [Y/N]  [N]

Memory  for Stack/Heap in K  .............. [96 ] (min 48K,  max

640K)

Use BIOS screen write method ...... [Y/N]  [N]

Download  Buffer Blocks ..................   [8  ]  (min 8,  max

240)

4. Qinstall - Misc. Parameters   Part 2

Add a few additional seconds to transfer timeout because QMODEM

uses  the serial port speed (9600) to compute timeout and  your

modem is running at a slower speed (2400).

Does your screen blink when Scrolling [Y/N]? [N]

Status window duration in Seconds [real] ... [0.5 ]

Additional Seconds added to Transfer Timeout [20  ] <-----

Are you using a PCjr? ...................... [N]

Connect Tones, up to 3 may be defined ...... [300 ] [350 ][400]

Download Completion Tone ................... [400 ]

Upload Completion Tone  .................... [300 ]

Default Terminal Emulation  ................ [B]

[A] TTY        [B] ANSI

[C] VT100      [D] TVI 925

5. QMODEM Phone Book

Set all entries to 9600 baud.

1 The Forbin Project, Home of QMODEM 1-319-233-6157  9600-8-N-1

2 Terrapin Station                   1-612-623-0152  9600-8-N-1

3.Multi-tech                         1-612=631-0922  9600-8-N-1

K. QMODEM NOTES

============

1. Be  sure  your modem is  turned on before calling QMODEM  3.1a.

Otherwise, QMODEM won't use CTS/RTS handshaking.

2. QMODEM  3.1a computes the efficiency of file transfers based on

the  serial port speed (9600 baud) rather than the modem  speed

(2400/1200/300  baud). This measures the efficiency of the  use

of  the serial port (irrelevant) rather than the efficiency  of

the  modem (what you really want). Sorry 'bout that. Maybe John

Friel will change that in a new release.

So,  don't be alarmed  by low efficiency numbers. These  aren't

real. Your modem really is working well.

In  the case below, a Y-modem transfer, the efficiency was 96%.

(You  must  multiply the  reported efficiency by 4 because  the

modem  connected at 2400 baud, the serial port was set to  9600

baud  and  9600 /2400 = 4.  If your connection is at 1200,  you

must multiply by 8.)

The average CPS figures are always accurate.

��[ Download File (receive) ]���������������������������͸

�Press [PgDn] to abort.                                  �

�Total Blocks received   : 6     Bytes/Blk :  577        �

�-- Long Block errors    : 0     Tot. Bytes: 6144        �

�-- Short Block errors   : 0                             �

�-- SOH errors           : 0   Average CPS :  231        �

�-- Complement errors    : 0    Efficiency : 24.0%       �

�-- Block number errors  : 0                             �

�-- Timeout errors       : 0                             �

�-- Resend Block errors  : 0                             �

�-- CRC errors           : 0                             �

�-- Last error message   :                               �

��������������������������������������������������������;

When  using an MNP level 5 connection, your efficiency may even

exceed   100%.  In  the  case  below,  a  reliable   compressed

connection was made at 2400 baud and Y-modem-G used to download

a  non-arc'd  text file.  The actual modem/line efficiency  was

186% because of compression. Again the CPS figures are correct.

This  example  also shows why  you need the serial port set  at

9600 baud. You could not have achieved any higher transfer rate

than 240 CPS (+- 20%) if the serial port had been set at 2400.

��[ Download File (receive) ]���������������������������͸

�Press [PgDn] to abort.                                  �

�Total Blocks received   : 18    Bytes/Blk :  743        �

�-- Long Block errors    : 0     Tot. Bytes: 18432       �

�-- Short Block errors   : 0                             �

�-- SOH errors           : 0   Average CPS :  447        �

�-- Complement errors    : 0    Efficiency : 46.5%       �

�-- Block number errors  : 0                             �

�-- Timeout errors       : 0                             �

�-- Resend Block errors  : 0                             �

�-- CRC errors           : 0                             �

�-- Last error message   :                               �

��������������������������������������������������������;

L. PROCOMM PLUS INSTALLATION INSTRUCTIONS

======================================

You  can call the set-up screen from PROCOMM Plus with Alt-S  or

from  DOS  with PCSETUP. Note  that some of the parameters  used

here are different from the installation program defaults.

1. PROCOMM PLUS SETUP UTILITY MODEM OPTIONS

The  initialization string needs  be only ATZ because you  have

stored your parameter settings with &W0.

A- Initialization command .. ATZ^M      <------

B- Dialing command ......... ATDT

C- Dialing command suffix .. ^M

D- Hangup command .......... ~~~+++~~~AT H0 S0=0 ^M

E- Auto answer command ..... AT S0=1 S2=255 E0 ^M

F- Wait for connection ..... 45  seconds

G- Pause between calls ..... 1   seconds

H- Auto baud detect ........ OFF         <------

I- Drop DTR to hangup ...... YES

(S2=255  disables the modem from responding to +++ from a remote

user  while  in HOST mode;  this would throw your modem  out  of

on-line  state.  After leaving HOST  mode you MAY have to  reset

this  to ATS2=43. It all  depends on which version of PROCOMM  +

you  have.  PROCOMM +  will reset it automatically whenever  the

modem is initialized.)

2. PROCOMM PLUS SETUP UTILITY MODEM RESULT MESSAGES

A- 300 baud connect message ...... CONNECT^M

B- 1200 baud connect message ..... CONNECT 1200

C- 2400 baud connect message ..... CONNECT 2400

D- 4800 baud connect message .....

E- 9600 baud connect message .....

F- 19200 baud connect message ....

G- No connect message 1 .......... NO CARRIER

H- No connect message 2 .......... BUSY

I- No connect message 3 .......... ERROR

J- No connect message 4 .......... NO DIALTONE

3. PROCOMM PLUS SETUP UTILITY TERMINAL OPTIONS

Turn on hardware flow control.

A- Terminal emulation ................ ANSI

B- Duplex ............................ FULL

C- Software flow control (XON/XOFF) .. ON      <-----

D- Hardware flow control (RTS/CTS) ... ON      <-----

E- Line wrap ......................... ON

F- Screen scroll ..................... ON

G- CR translation .................... CR

H- BS translation .................... DESTRUCTIVE

I- Break length (milliseconds) ....... 350

J- Enquiry (ENQ) ..................... OFF

4. PROCOMM PLUS PHONEBOOK

Set all the entries in the phone book to 9600 baud.

M. TELIX 3.00 INSTALLATION INSTRUCTIONS

====================================

Call the configuration menu with alt-O.

�͵ Configure Telix ������������͸

�                                �

� Change which default settings? �

�                                �

�      Screen and color          �

�      Terminal options          �

�      General options           �

�      Modem and dialing         �

�      Filenames and paths       �

�      Ascii transfers           �

�      Protocol options          �

�      Kermit transfers          �

�      Comm port setup           �

�      Write setup to disk       �

�      Exit                      �

�                                �

��������������������������������;

On  the terminal options  screen, set XON/XOFF flow control  ON

and set CTS/RTS hardware flow control ON.

�͵ Terminal options ��������������������������������������������͸

�                                                                 �

� A - Baud 9600    Parity None    Data length 8    Stop bits 1    �

� B - COM1                                                        �

�                                                                 �

� C - Default terminal type ........... VT102                     �

� D - Status line ..................... Bottom                    �

� E - Local echo ...................... Off                       �

� F - Add Line Feeds after CRs ........ Off                       �

� G - Strip high bit (incoming data) .. Off                       �

� H - Received Backspace destructive .. On                        �

� I - XON/XOFF software flow control .. On                        �

� J - CTS/RTS hardware flow control ... On                        �

� K - DSR/DTR hardware flow control ... Off                       �

� L - Compuserve Quick B transfers .... Off                       �

� M - Zmodem auto-downloads ........... On                        �

�                                                                 �

�                                                                 �

� Change which setting?       (Return or Esc to exit)             �

�����������������������������������������������������������������;

Note there is no space between DIAL and TONE.

�͵ Modem and dialing parameter setup ����������������������������͸

�                                                                  �

� A - Init string ......... ATZ S0=0 S2=43 E1 ^M~                  �

� B - Dialing prefix 1 .... ATDT                                   �

� C - Dialing prefix 2 .... ATDT                                   �

� D - Dialing prefix 3 .... ATDT                                   �

� E - Dialing suffix ...... ^M                                     �

� F - Connect string ...... CONNECT                                �

� G - No connect strings .. NO CARRIER             BUSY            �

�                           ERROR                  NO DIALTONE     �

� H - Hang-up string ...... ~~~+++~~~ATH0^M                        �

� I - Auto answer string .. ~~~+++~~~ATS0=1 S2=255 E0 ^M~          �

� J - Dial cancel string .. ^M                                     �

�                                                                  �

� K - Dial time ........... 45                                     �

� L - Redial pause ........ 1                                      �

� M - Auto baud detect .... Off                                    �

� N - Drop DTR to hangup .. On                                     �

�                                                                  �

�                                                                  �

� Change which setting?       (Return or Esc to exit)              �

������������������������������������������������������������������;

Set all entries in the dialing directory to 9600 baud.

N. TESTING

=======

Test your settings by dialing the MultiTech Bulletin Board.

612-631-0922.  Read  the messages  to test data compression  of

text   files.  Try  downloading  to  test  your  file  transfer

protocols.

O. TESTED CONFIGURATION

====================

These  settings  have  worked  reliably for my system:  an  old

fashioned  IBM PC (ROM BIOS  10/27/82), an AST six pack  serial

port,  and a MultiTech 224EH with ROM 1 version # = 607 and ROM

2 version # = 506.

P. REPORTED PROBLEMS

=================

1. A  small bug in QMODEM  3.1a: when receiving text non-stop  and

you  go  into the scrollback  buffer (up arrow) and you are  in

reliable  mode  (&E1)  and  the  buffer fills  up  (about  6200

characters), then QMODEM locks up the machine. The bug does not

occur  when you are out  of reliable mode (&E0). This bug  does

not apply to file transfers.

PROCOMM  Plus 1.0 does not  have this problem in the  redisplay

buffer  (Alt-F6). It stops when its buffer fills up and  begins

receiving more data when you exit the redisplay screen.

2. There is a bug in the USR HST 9600 modem, a modem found on many

BBS's.  This bug causes  uploads involving Ymodem, Sealink  and

Zmodem to from your MT224EH to the HST to fail.

Connected  at MNP level  5, the uploading software reports  bad

blocks  and  aborts after  about  5 tries. At that  point,  the

keystrokes  sent to the  remote are not echoed back  correctly.

Sometimes  two  characters are  returned for one keystroke  and

anyway  the characters are never correct. The only solution  is

to disconnect and re-dial.

The problem also occurs when using MNP level 4 (MT224EH command

&E14),  in that the upload aborts; however, after the abort you

can  continue  to  talk  with  the  BBS  and  do  not  have  to

disconnect.

This bug was introduced into HST modems shipped between Dec '87

and  Feb  '88 but has  now been corrected. The solution to  the

problem is for the HST owner to get an updated ROM from USR.

A  workaround for for the HST  owner is to disable MNP level  4

and higher, or to reduce the Xmit buffer size to 255 characters

or fewer.

A  workaround for the MT224EH owner is disable MNP (&E0)  prior

to dialing into a BBS with a buggy HST.

(This information from Tom Soderberg, sysop at Multitech.)

One HST which has these problems was set as follows:

Modem init string  AT&FE0H0M0X6&H1&B1S2=255S7=45S0=2

ATI7 returned:

Product type             External

Custom chip              U100

Clock freq               14.976

Options                  Standard

Supervisor date 11/25/87

Supervisor rev  4

Transmitter rev 2

Receiver rev    2

Q. ADDENDUM - DSZ SETTINGS

=======================

I'm  not a user of DSZ but I downloaded some old  documentation

and  tested the following receive command. It worked  correctly

with  QMODEM 3.1A. The "handshake on" appeared to be  necessary

to get DSZ to work with CTS/RTS flow control. I have not tested

the send command.

for receiving:

DSZ handshake on rz

for sending:

DSZ handshake on sz %3 %4 %5 %6 %7 %8 %9

R. ADDENDUM - MT224EC HARDWARE SWITCH SETTINGS

===========================================

I  don't  have the MT224EC  (the internal 2400 modem) but  have

been informed that the switches should be set as follows:

Bank 1:

#1 Carrier Detect normal

#2 Set to desired COM: port

Bank 2:

#1  UP auto-reliable

#2  UP hardware flow control

#3  DOWN   "        "

#4  DOWN MultiTech responses

###################################