BOOT.COM

Karl Brendel

316-241-2340

8 April 1985

For the IBM PC-AT only!

This routine allows a warm boot to be called from a batch file or the

keyboard.  To run the program, type  BOOT  [ENTER]. It bypasses the RAM check

which is made at the beginning of a cold boot and when apparently similar

routines such as IPL.COM are run.

This routine violates the IBM philosophy of accessing the BIOS.  Rather

than doing that through INT calls, this sets a value into a specific memory

address, then jumps to a specific ROM address.  The result of this means of

accessing the BIOS is that it may not work with ROMs other than those

identical to mine.  To determine if yours is identical to mine, use DEBUG to

dump addresses f000:0000 thru 000d.  You should see 66118811002289.

If that is not what you find, then the situation may get a little messy.

Remove any memory-resident applications such as SideKick, Newkey, etc., from

RAM (the best was is to rename your autoexec file and do a Ctrl-Alt-Del)  Still

using DEBUG, do this:

a cs:100<    (< means enter)

int 16<

<

t=100 2<

Look at the display of the last trace output line, which should appear

something like

F000:XXXX       STI

Add to XXXX, 201 hex, giving (say) XYYY.  Do

d f000:XYYY L b

The bytes shown (not their character representations, shown at the right of

the DEBUG dump display) should be

F6 06 17 00 04 74 31 3C 53 75 2D

If they are, then you may use this routine with no patches.

If you have made it this far and do not feel that you have a compatible

BIOS ROM, then you have a couple of alternatives:  You may take this stuff and

trash it.  You may use this and a copy of the AT DOS 3.0 BIOS and revise the

routine to support your ROM.  You may use a public domain routine such as IPL

which will allow you the same operation, but will not bypass the RAM check.

Whatever you do, I wish you luck and hope that this has at least stimulated

some thought.

Boot functions by jumping into the middle of the BIOS Keyboard IO

routines.  It first stores a value into the keyboard flags register to

simulate the presence of the Ctrl-Shift combination.  It then places the code

for the Del key into the AL register and makes a direct jump to the portion of

the BIOS where a check is made for Ctrl-Alt-Del.

Note that the actual code is only 17 bytes long, although the file you

downloaded is somewhat longer.  That has to do with the structure of LBR files

and transmission protocols.  You may use DEBUG to eliminate the excess after

the 17th byte.

The assembler code:

B84000        MOV     AX,0040              SET UP DS

8ED8          MOV     DS,AX

C606170004    MOV     BYTE PTR [0017],04   PUT FLAG FOR CTRL-SHFT

IN KB_FLAG

B053          MOV     AL,53                PUT DEL KEY CODE IN AL

EAC93100F0    JMP     F000:31C9            JUMP TO WHERE TEST IS MADE