PRINTER.DOC                    Ver 1.1

John Petrey

(c) 1983

This  is  the documentation file for the PRINTER.COM  program,  a

program that allows one to easily use the special print functions

for  their printer (any make).   PRINTER.COM is run by typing  in

PRINTER at the DOS prompt or directly from Wordstar using the "R"

(run a program) command.

PRINTER.COM  is  used to set up your printer to make use of  it's

special print functions (compressed print,  double strike  print,

italics print,  etc.)  For example,   you may have a printer that

normally only prints 80 characters on a line,  which is fine most

of  the  time.   However,  you may find times when you'd like  to

print more than 80 characters on a line.  You can use PRINTER.COM

to tell your printer to use compressed print.    Likewise, if you

normally use continuous form paper,  then you want the printer to

stop printing  when it runs out of paper.   But when you want  to

use  single sheet paper,  the printer stops because it thinks  it

doesn't have any paper in the printer.   PRINTER.COM can tell the

printer to temporarily ignore the paper out sensor so you can use

single sheet paper.   Whatever your printer has the capability to

do, you can easily do it with PRINTER.COM.

To use PRINTER.COM you must set up a data file for PRINTER.COM to

read  so it knows what special print functions you want  to  use.

You  can enter up to ten different special print features in  the

data  file.   The  data file must be named PRINTER.DAT  and  must

conform  to  a specific format so PRINTER.COM can understand  it.

When you run  PRINTER.COM,  it looks for the  PRINTER.DAT file on

the default disk drive unless you tell  PRINTER.COM  that the DAT

file is located on some other disk drive.    You tell PRINTER.COM

the drive the  DAT file is located on by typing in the disk drive

letter after you type in PRINTER at the DOS prompt.  For example,

if the  default  drive is  A  and  PRINTER.COM  is on drive B and

PRINTER.DAT is on drive C, you would type in  B:PRINTER C  at the

DOS A> prompt so DOS would know to look on drive B for PRINTER.COM

and so PRINTER.COM would know to look on drive C for PRINTER.DAT.

The  data file (PRINTER.DAT) must contain a description (literal)

of  the special print features you want to be available  and  the

printer  control  codes  for each  special  print  feature.   The

control  codes  are listed in your printer's user manual and  are

likely found in the back of the manual as an appendix.  A control

code  is  nothing  more  than  a unique  code  that  the  printer

recognizes as a command to change to a special print mode.

Let's assume you have an Epson MX printer and want to be able  to

set up the printer for compressed print.   According to the Epson

manual  (appendix B in my Epson manual) the control code to  turn

on  compressed print is 15 (the ASCII character 15).  To  set  up

your  PRINTER.DAT  file for compressed print on an Epson  printer

you would need an entry like the following:

Compressed Print$015000

Page 1

The  first  part  of the entry is what you want  printed  on  the

screen,  Compressed Print in this case.  Following the literal is

a  dollar sign ($) which tells PRINTER.COM where the end of  your

literal is and where the beginning of the codes are that you want

sent  to  the  printer to turn on  that  special  print  feature.

Following the dollar sign are six digits which to PRINTER.COM are

really  two three-digit numbers.   The first three digits in this

case  are  015  which  will be sent to the  printer  to  turn  on

compressed print.   The last three digits (000 in this case) will

not  be  sent  to the printer because  it  is  000.   PRINTER.COM

expects two three-digit numbers,  so in this case we had to enter

000   as   one  of  the  two  three-digit  numbers   to   satisfy

PRINTER.COM's  requirement  for two  three-digit  numbers.   Many

special print functions require two control codes be sent to  the

printer  in order to set up a special print function,  so that is

why PRINTER.COM wants two-three digit numbers.   For example,  on

the Epson Printer,  to set up Italics print you need to send  the

printer  a  ESC (escape) and a 53 (according to my Epson  Printer

manual,  Appendix  B).   You would enter  your  literal,  Italics

Print,  followed by a dollar sign followed by 027053.  027 is the

ASCII code for Escape  and  053 is the code  for  Italics.  Thus,

the entry in the PRINTER.DAT file would look like:

Italics Print$027053

Up  to  ten different special print functions can be  entered  in

the PRINTER.DAT file and each should be on separate  lines.   The

last  entry  should be followed by an ampersand  (&)  immediately

after  the last digit of the last code.  The ampersand (&)  tells

PRINTER.COM  when  it has  reached the end of your  special print

features.   For  example,  if you only  had two  entries  in your

PRINTER.DAT file it might look like:

Compressed Print$015000

Italics Print$027053&

Remember,  1) you can have up to 10 entries in PRINTER.DAT

2) each entry is comprized of:

a. literal followed by dollar sign

b. followed  by two three-digit codes  (decimal

ASCII value - not hex)  that  are to be sent

to the printer   (Refer  to  your  printer's

manual for the  control codes  to set up the

special print features you want to use.)  If

your  printer  manual says to  send a  alpha

character to the  printer,  you can find out

what  ASCII  code should be sent by refering

to the IBM-PC BASIC manual appendix G.

3) The last  digit  of last code is followed  by  an

ampersand  (&)  so  PRINTER.COM knows when it has

reached the last of your special print features.

Page 2

The  PRINTER.DAT  file  can be created with  any  editor  (Edlin,

Wordstar,  directly from the console, etc.).  Just be sure if you

use  Wordstar  that you create/edit the PRINTER.DAT file  in  the

non-document  mode.   A  sample PRINTER.DAT file  (for  an  Epson

printer) is provided as an example.

PRINTER.COM  can  be  run  from Wordstar by using the  R  (run  a

program) command from the Wordstar Menu.  For example, assume you

just finished editing a document and have saved it.   You  should

now  be at the Opening Menu where you normally would strike P  to

begin printing the document.   Before printing the document,  you

can  strike  R for run a program at which time Wordstar will  ask

you for the name of the program you wish to run.  Type in PRINTER

and  strike the enter key.   Note:  if PRINTER.COM is not on  the

logged  (default) disk drive you'll need to prefix  PRINTER  with

the  drive  specification  of the drive where Wordstar  can  find

PRINTER.COM  (such as A:  for drive A).  If PRINTER.DAT is not on

the default disk drive, you'll need to specify the drive where it

can be found after entering PRINTER.  For example, if the default

drive is B: and both PRINTER.COM and PRINTER.DAT are on drive A:,

then you'll need to enter   A:PRINTER A   so  Wordstar  will know

where to look for PRINTER.COM  and PRINTER.COM will know where to

look for  PRINTER.DAT.   After you've entered this, Wordstar will

run the PRINTER.COM program which will prompt you for the special

print  functions  you  set  up in your  PRINTER.DAT  file.   Upon

exiting  PRINTER.COM,   you will automatically return to Wordstar

where  you  can now strike P to begin printing your document with

the special print function(s) you selected in effect.

Listed below is a sample PRINTER.DAT file for an EPSON printer.

Set Printer to Power Up Mode$027064

Compressed Print$027015

Italics Print$027052

Emphasized Print$027069

Double Strike Print$027071

1/8th Inch Spacing$027048

Turn on Paper Out Sensor$027057

Turn off Paper Out Sensor$027056

Double Width Print #1$027087

Double Width Print #2$001000&

Note: To use Double Width Print,  both Double Width options

in the above sample must be entered.  This is because

the Epson printer needs three control codes to set up

Doulbe Width print.   Since PRINTER.COM can only have

two control codes per entry in the  PRINTER.DAT file,

two entries in  PRINTER.DAT  are required to send the

three control codes the Epson printer wants.

#